COTS Components Aurora

1	General Information	6

1.1	Document scope, ownership and responsibility	6

1.2	Component short description	6

1.3	Reasons for development	8

1.4	Usage and position in vehicle	8

1.5	Variants	10

1.6	Geometrical limitations or Interfaces	10

1.6.1	Interfaces – Design review	10

1.7	Supplier Drawings	11

1.7.1	General 2D Drawing Requirements	11

1.7.2	Criticality	11

1.8	Functions	11

1.9	General test conditions	11

1.9.1	Evidence of test results	13

1.10	Verification overview	14

1.10.1	Test sequence	14

1.10.2	Test report content	15

1.11	Test preparations	15

1.11.1	Initial and final status of component	15

1.11.1.1	Initial function test, normal climate	16

1.11.1.2	Initial function test, cold climate	16

1.11.1.3	Initial function test, hot climate	16

1.11.1.4	Final function test, normal climate	16

1.11.1.5	Final function test, cold climate	16

1.11.1.6	Final function test, hot climate	16

1.11.1.7	Post-test analysis	16

1.11.2	Monitoring	17

1.11.2.1	Monitoring – Design review	17

2	Quality and Uptime	17

2.1	Reliability/Durability	17

2.1.1	Reliability expectancy	17

2.1.1.1	Reliability expectancy - Design review	17

2.1.1.2	Reliability expectancy – Test	17

2.1.2	Vibration	18

2.1.2.1	Random vibration & temperature cycling – Long duration - Test	18

2.1.2.2	Random vibration - Short duration, 1-axis – Test	20

2.1.2.3	Random vibration – Short duration, 3-axis – Test	21

2.1.2.4	Sinus vibration & temperature cycling – Test	22

2.1.2.5	Mechanical shock – Test	23

2.1.2.6	Multi-axial rig / shake – Test	23

2.1.2.7	AET – Test	23

2.1.3	Dust intrusion	23

2.1.3.1	Degrees of protection (1st IP code) – Test	24

2.1.3.2	Dust – Test	24

2.1.4	Moisture intrusion	24

2.1.4.1	Permeability and operational – Test	24

2.1.5	Water intrusion	25

2.1.5.1	Ice water shock, splash – Test	25

2.1.5.2	Ice water shock, Submersion – Test	25

2.1.5.3	Degree of protection (2nd IP code) – Test	25

2.1.5.4	Salt spray – Test	26

2.2	Atmospheric environment lifetime	26

2.2.1	Operating temperature	26

2.2.1.1	Cold temperature wake-up – Test	26

2.2.1.2	High temperature, operation – Test	26

2.2.2	Sunshine	26

2.2.2.1	Light exposure (exterior) – Test	27

2.2.3	Climate cycling	27

2.2.3.1	Stabilization time – Test	27

2.2.3.2	Rapid change of temperature (thermal shock) – Test	28

2.2.3.3	Temperature cycling – Test	28

2.2.3.4	Temperature step – Test	29

2.2.4	Process oven temperature	30

2.2.4.1	Storage temp aftermarket paintshop – Test	30

2.2.5	Moisture	30

2.2.5.1	Composite temperature/Humidity cycling – Test	30

2.2.5.2	Damp heat, steady state – Test	30

2.2.6	Air pollution	31

2.2.6.1	Corrosion test with flow of mixed gases – Test	31

2.3	Quality impression	31

2.3.1	Surface finish	31

2.3.2	QI Solidity	32

2.3.2.1	QI Solidity – Evaluation	32

2.3.3	Corrosion	32

2.3.3.1	Corrosion test on complete vehicle – Test	32

2.3.4	Scratch resistance	32

2.3.4.1	Scratch resistance by Erichsen pen – Test	32

2.3.4.2	Scratch resistance (abrasive paper) – Test	33

2.3.5	Chemicals	33

2.3.5.1	Resistance to chemical loads – Test	33

2.3.5.2	Ageing in fluids (plastic/rubber) – Test	34

2.3.6	Material and composite materials properties	35

2.3.6.1	Material properties – Documentation	35

2.4	Maintainability	35

2.4.1	Design for maintainability	35

2.4.1.1	Design for maintainability – Design review	36

3	Safety	36

4	Driver (operating) environment	36

5	Environment – Materials and environmental impact	36

5.1	Design for recycling	36

5.1.1	Design for recycling – Design review	37

5.2	Material selection	37

5.2.1	Material selection – Design review	37

5.3	Substances that shall not be present in product	38

5.3.1	Substances that shall not be present in product – Reporting	38

5.4	Emissions and odours	38

6	Driving performance	38

7	Transport effectiveness	38

7.1	Component weight	38

7.1.1	Component weight – Documentation	39

8	Vehicle regulation and Regulative conditions	39

8.1	Vehicle regulations and certification	39

8.1.1	Vehicle regulations and Certification (xxx) – Design review	39

9	Electrical & Electronics	39

9.1	E&E mechanical properties	39

9.1.1	Silicone-free	39

9.1.1.1	Silicone- free – Design review	40

9.1.2	Sulphur-free rubber	40

9.1.2.1	Sulphur-free rubber – Design review	40

9.1.3	Lead-free	40

9.1.3.1	Lead-free – Design review	40

9.1.4	Connectors	41

9.1.4.1	Connector – Design review	41

9.1.5	Electrical wiring	41

9.1.6	Pin description	41

9.1.6.1	Pin description – Documentation	42

9.1.7	Printed Circuit Board (PCB) assembly	42

9.1.7.1	PCB assembly – Design review	42

9.1.8	PCB Conformal Coating	43

9.1.8.1	PCB conformal coating –Design review	43

9.2	E&E Production requirements	44

9.2.1	End Of Line functional – Test	44

9.2.1.1	EoL – Design review	44

9.2.2	In-Circuit Test	44

9.2.2.1	ICT – Design review	44

9.3	Electrical Properties Low Voltage	44

9.3.1	Voltage rating	44

9.3.1.1	Slow decrease and increase of supply voltage – Test	47

9.3.1.2	Momentary drop in supply voltage – Test	47

9.3.1.3	Over-voltage – Test	48

9.3.1.4	Power supply interruption – Random test	48

9.3.1.5	Reset behaviour at voltage drop – Test	49

9.3.1.6	Reversed voltage – Test	49

9.3.1.7	Superimposed alternating voltage – Test	50

9.3.1.8	Superimposed pulse voltage – Test	50

9.3.2	Power consumption	51

9.3.2.1	Direct current supply voltage and current consumption – Test	51

9.3.3	Grounding	51

9.3.3.1	Grounding – Test	52

9.3.4	Open-circuit protection	53

9.3.4.1	Open-circuit protection – Test	53

9.3.5	Short-circuit protection	53

9.3.5.1	Short-circuit protection – Test	53

9.3.6	HW start up time	54

9.3.6.1	HW start up time – Test	54

9.4	Electromagnetic Compatibility (EMC)	54

9.4.1	Conducted Transient emission	55

9.4.1.1	Conducted transient emission Test case 1	56

9.4.1.2	Conducted transient emission Test case 2	57

9.4.1.3	Conducted transient emission Test case 3	57

9.4.1.4	Conducted transient emission Test case 4	57

9.4.1.5	Conducted transient emission Test case 5	58

9.4.1.6	Conducted transient emission Test case 6	58

9.4.2	Conducted RF emission – Voltage method	58

9.4.2.1	Conducted RF emission	59

9.4.3	Conducted transient susceptibility on power supply leads and I/O:s connected to power supply	59

9.4.3.1	Test pulse 1	61

9.4.3.2	Test pulse 2	62

9.4.3.3	Test pulse 3	63

9.4.3.4	Test pulse 4	64

9.4.3.5	Test pulse 5	65

9.4.4	Conducted transient susceptibility on signal leads	66

9.4.4.1	Conducted transient susceptibility on signal leads fast	66

9.4.4.2	Conducted transient susceptibility on signal leads Slow	66

9.4.5	Power supply quality	66

9.4.5.1	Power supply with battery	67

9.4.5.2	Power supply without battery	68

9.4.5.3	Immunity to micro power cuts	69

9.4.5.4	Immunity to high-voltage supply	69

9.4.6	Radiated emission CV (Complete Vehicle)	70

9.4.6.1	Radiated emissions measured at the foot of the antenna – Test	70

9.4.6.2	Radiated emissions measured in accordance with ECE R10 - Test	72

9.4.7	Radiated emission	74

9.4.7.1	Radiated emission ALSE	74

9.4.8	General - Radiated susceptibility	77

9.4.8.1	Radiated susceptibility CV (Complete Vehicle) - Test	79

9.4.8.2	Radiated susceptibility component - Test	79

9.4.8.3	Radiated susceptibility component test -Far field Radar	80

9.4.8.4	Radiated susceptibility component test -BCI	80

9.4.8.5	Simulated portable transmitter	81

9.4.9	Immunity to electrostatic discharge	81

9.4.9.1	Immunity to ESD Handling	81

9.4.9.2	Immunity to ESD Powered	82

9.4.9.3	Immunity to ESD Complete vehicle	83

9.4.9.4	Pre damage ESD	84

9.4.10	Immunity to low-frequency magnetic fields	84

9.4.10.1	Immunity to low-frequency magnetic fields	85

9.4.11	Emission of low-frequency magnetic fields	85

9.4.11.1	Test setup complete vehicle	85

9.5	Network/communication	86

9.6	Diagnostics and control	86

9.6.1	Diagnostics – Design review	86

10	Manufacturing and logistics	86

10.1	Part handling by GTO/AM	86

10.2	Component design for assembly	86

10.2.1	Component design for assembly – Design review	88

10.3	Component design for logistics	88

10.3.1	Component design for logistics – Design review	89

11	Aftermarket	90

11.1	Design for aftermarket	90

11.1.1	Design for aftermarket – Design review	90

12	Cybersecurity	90

12.1	Cybersecurity Engineering	90

12.1.1	Cybersecurity engineering – Design review	90

12.2	Cybersecurity supply chain management	90

12.2.1	Cybersecurity engineering - Design review	90

13	Other requirement areas	91

13.1	Marking	91

13.1.1	Marking – Design review	91

14	References	92

14.1	Reference documents	92

14.2	Abbreviations and terms	93

15	Changelog	95

15.1	New items	95

15.2	Updated items	95

15.3	Deleted items	95

16	Approved Deviations	95

# General Information

## Document scope, ownership and responsibility

TR information	INF 54523319-1 v2

This Technical Requirement specifies the product requirements and related verification methods for the Commercial off-the-shelf (COTS) components provided by Aurora.

Volvo Autonomous Solutions, a unit of the Volvo Group is the owner of this document.

All parties involved are welcome to use their expertise to suggest updates and improvements.

The relevant Volvo Group engineering groups have the main responsibility to initiate, review and approve this document.

## Component short description

TR information	INF 54523319-2 v3

The COTS components provided by Aurora consist of the following:

- 2x combined Multi-band (LTE/Wi-Fi) antennas (Representative image only, not exact design)

- 2x GNSS antennas (Representative image only, not exact design)

- 1x Siren-detection microphone (Representative image only, not exact design)

- 1x Loudspeaker (Representative image only, not exact design)

## Reasons for development

TR information	INF 54523319-3 v2

The COTS components are part of a key system that will provide self-driving capabilities to trucks, when combined with an autonomy-enable truck platform.

## Usage and position in vehicle

TR information	INF 54523319-4 v3

The antennas are located on the back of the cab. The microphone is integrated into the overhead panel.

The Loudspeaker is bolted on the TMS mounting rack using a adapter bracket.

Owing to the specific Operational Design Domain (ODD) and use-cases for this generation of product, certain parameters and standard Volvo requirements may have been adjusted to account for this – including levels of expected vibration, shock, and operational temperature. It is understood that any alteration to the intended ODD or use-cases for the product shall drive a review of all requirements in this document, to confirm applicability to the new ODD or use-cases.

For the purposes of this Technical Requirements document, the ODD and use-case are understood to be:

(For discussion during RTS (Review of Technical Specifications)  review)

| Parameter | Specific | In Scope (Autonomous) | In Scope (all other vehicle operations) |
| --- | --- | --- | --- |
| Drivable Surface Type | Asphalt, Concrete | True | True |
|  | Brick, Cobblestone, Sett | False | True |
|  | Dir, Grass, Gravel | False | True |
|  | Tar&chip pavement | False | True |
|  | Railroad Tracks | False | True |
| Road Surface Object | Barrier_Curb | True | True |
|  | Botts_Dotts | False | True |
|  | Mountable_Curb | False | True |
|  | Speed_Bump | False | True |
|  | Raised_Pavement_Marker | True | True |
|  | Rumble_Strip | True | True |
|  | Manhole_Cover | True | True |
| Environmental | Ambient Temperature | -10/+44C operating | -40/+60C tolerate, survive |
|  | Ambient Relative Humidity | 10% ~ 90% |  |
| Use-case | Hub-to-Hub; improved surface streets and US Interstate system, |  |  |

Reference:  Volvo Trucks MVP Operational Design Domain Specification / Aurora doc 500-001581-01.

## Variants

TR information	INF 54523319-5 v2

The COTS components provided by Aurora will not have any variants.

## Geometrical limitations or Interfaces

HWC requirement	REQ 54523319-1 v3

No changes to the geometry, mounting provisions, or interfaces to any of the of the COTS components shall be made without coordination between Volvo and Aurora.

The interfaces between the COTS and the vehicle platform are defined in several documents including:

Autonomous Driving Partner ICD: 300-014436-01_REV-1.1_ICD, ANTENNA, GNSS, AG1.0_PDF

Autonomous Driving Partner ICD: 300-014437-01_REV-1.1_ICD, ANTENNA, MULTI-BAND, AG1.0_PDF

Autonomous Driving Partner ICD: 300-018768-01_REV-1.1_ICD, SPEAKER, AG1.0_PDF.pdf

### Interfaces – Design review

Verification method	VM 54523319-1 v2

The fulfilment of the requirements is verified by a review of the component design with the concerned stakeholders.

## Supplier Drawings

TR information	INF 54523319-6 v2

Supplier drawings are important for conveying information between Aurora and Volvo and will be used for documentation of the Aurora components within the Volvo systems.  The information in the drawings (both 2D and 3D) will be used for securing that the components are properly integrated – including in terms of geometry, physical properties, and process.

### General 2D Drawing Requirements

HWC requirement	REQ 54523319-2 v2

Aurora shall supply a manufacturer datasheet / specification sheet for each of the COTS parts which are the subject of this TR, indicating (at a minimum):

- All installation interfaces (interfaces between Aurora components and the vehicle platform) shall be fully documented either on the drawing, or on a referenced document (e.g., Aurora Interface Control Document, ICD)

- Materials and surface treatment shall be stated

- Weight (in kg) with tolerance shall be stated

- Administrative information:  Supplier name, supplier document number (with revision), change log.

Verification method	VM 54523319-2 v2

The fulfilment of the requirements shall be verified by a Drawing Review with the concerned stakeholders.

Pass criteria

Documented/stated compliance with requirements.

### Criticality

HWC requirement	REQ 54523319-3 v2

The COTS components which are the subject of this TR shall be analyzed with respect to criticality, according to Volvo STD 105-0007.

Any characteristics which are considered to be of criticality CC (Critical Characteristics)  or SC (Significant  Characteristics) according to Volvo STD 105-0007 shall be disclosed to Volvo. These CC/SC declarations must be associated with the product documentation when stored in the Volvo systems.

Verification method	VM 54523319-3 v2

The fulfilment of the requirements shall be verified by a Design Review with the concerned stakeholders.

Pass criteria

Documented/stated compliance with requirements.

## Functions

TR information	INF 54523319-7 v2

This TR is not intending to define or describe any of the functions which the COTS components will perform.

This document is intended to describe the conditions (and tests to ensure the COTS components will survive those conditions), when operating as a critical element of an L4 Autonomous Transport Solution.

## General test conditions

TR information	INF 54523319-8 v3

Whether Aurora chooses to perform any of the tests in this TR themselves, or whether they choose to have an outside test house perform the tests, then the following General Test Conditions shall apply to all tests:

Conditioning and general test conditions

According to STD 400-0001 Standard atmospheres for conditioning and testing.
The test samples shall be conditioned for 24 h before testing is performed. 
Standard atmosphere class 1.

In between the heat treatment, ageing and climate-cycling stages, the part shall be conditioned in accordance with the section “Conditioning”.

Applicability	
The requirements apply to the complete component, unless otherwise stated in relevant sections in this document.

Test samples 	
Testing shall be performed on complete components, unless otherwise stated. When these are not used, special test pieces must be made. All tests are applied on standalone Aurora components without any Volvo-supplied covers.

Complete-vehicle verification and component requirements

Component testing is a preparatory step in the complete vehicle or system verification.

The component shall fulfill component pass criteria, even when installed and tested in complete-vehicle or system-level tests.
If the component fails the complete-vehicle or system verification, the component design will need to be improved and the requirements in the component verification will need to be updated.

No component is allowed to fail during testing. A failure is defined as an event or occurrence outside the specification. Components must not be replaced during testing / the test sequence.

Testing shall be performed on production-grade material manufactured in a production-grade process. Any deviation shall be approved by Volvo VAS. The run rate for each process step shall be at least the run rate agreed. However, the entire process flow does not have to be in place. That is, the final production layout can be completed at a later stage. The only deviation from this is when consecutive process steps need to be carried out in sequence with the correct lead times (e.g. the process of coating and curing the coating must be done in a production-grade manner).

If production is performed in several cavities, each cavity shall be represented in the test population. Each variant of the product shall be represented by the number of samples given in the test overview, unless otherwise discussed with and accepted by Volvo.

Test conditions for electrical and electronic equipment

All testing shall be performed with the connectors and harness installed, unless otherwise stated in this TR. This includes mating the connectors and harness up to at least the first clamping point. For components with a metallic housing, the casing shall be connected to ground in same manner as in the truck. The component shall be installed the same way (fixing points, flange screws, etc.) and with the same angle as in the truck, based on the information provided by Volvo. The test objects shall be powered and connected to the load circuit during testing, in accordance with the operating mode definition, unless otherwise stated in the test specification.

The pass criteria description often has a reference to the Functional Status Classification (FSC).

Operating mode codes

Definition of codes for operating modes – see ISO STD 16750-1, chapter 5

| Code | Definition |
| --- | --- |
| 1 | No voltage is applied to the component. |
| 1.1 | Not connected to wiring harness. |
| 1.2 | Connected to wiring harness simulating vehicle installation. |
| 2 | The component is electrically operated with test voltage, UB, as in a vehicle with generator off and with all electrical connections made. |
| 2.1 | System/component functions are not activated. |
| 2.2 | Systems/components with electric operation and control in typical operating mode. |
| 3 | The DUT is electrically operated with test voltage, UA as in a vehicle with generator running and with all electrical connections made. |
| 3.1 | System/component functions are not activated. |
| 3.2 | Systems/components with electric operation and control in typical operating mode. |

Functional status class (FSC)
Definition of the FSC classes – see ISO STD 16750-1, chapter 6

| Code | Definition |
| --- | --- |
| FSC A | All functions of the component perform as designed (to specification), both during and after exposure to normal operating conditions or a disturbance. |
| FSC B | All functions of the component perform as designed during exposure; however, one or more functions may go beyond the specified tolerance. All functions automatically return to within normal limits after exposure is removed. Memory functions shall remain FSC A. The component shall not output any false actuation signals, erroneous serial data messages, false fault codes, or other erroneous Input/output (I/O) commands or states when the environmental loads are outside of the range defined for FSC A. For FSC B, all deviations (test case by test case) from FSC A must be agreed between the supplier and Volvo before C-release. If applicable, add a list of functions that must be maintained during tests where FSC B is used. |
| FSC C | A function of the component does not perform as designed during exposure but automatically returns to normal operation after exposure is removed. The component shall not output any false actuation signals, erroneous serial data messages, false fault codes, or other erroneous I/O commands or states when the environmental loads are outside of the range defined for FSC A. |
| FSC D | A function of the component does not perform as designed during exposure and does not return to normal operation until exposure is removed and the component is reset by a simple operator action. The component shall not output any false actuation signals, erroneous serial data messages, false fault codes, or other erroneous I/O commands or states when the environmental loads are outside of the range defined for FSC A |
| FSC E | One or more functions of the component/system do not perform as designed during and after exposure, and cannot be returned to normal operation without repairing or replacing the component. For Volvo components, this is not allowed except for short-circuits s to non-short-circuit-protected signal returns or Vcc-supplied sensors (Vcc=5V or lower) with short to battery test voltage levels above 16V. No thermal event is allowed for FSC E. |

### Evidence of test results

TR information	INF 54523319-9 v3

For the Commercial-Off-The-Shelf (COTS) components which Aurora will be providing, verification of the requirements within this section can be satisfied by one of three ways:

- Aurora (or outsourced test house) conduct the TR-listed test (or an equivalent test, to be agreed between Volvo and Aurora) for each of the COTS components, and provide the requested proof of compliance, or

- If Aurora determines that no testing is necessary  then they shall state the rationale that lead to that determination.

During the Review of Technical Specifications (RTS) – Aurora shall indicate their intended path for         each COTS component, for each required test.

- Aurora provides evidence that each of the COTS components have been tested and/or certified by the manufacturer to pass the TR-listed tests (or an equivalent test, to be agreed between Volvo and Aurora).
(If this is the chosen path, it is Aurora’s responsibility to verify any manufacturer claims of compliance, since Aurora has chosen that supplier)

During the Review of Technical Specifications (RTS) – Aurora shall indicate their intended path for each COTS component, for each required test.

## Verification overview

### Test sequence

TR information	INF 54523319-10 v2

TR tests that shall be performed in a specific sequence are documented below. The tests that are crossed-out are not required for the component. The following test sequences shall be discussed with stakeholders during RTS (Review of Technical Specifications) and TRA (Test Requirements Analysis).

Note: The tests which are clarified in the text of the TR needs to be done in the sequence specified in the figure. If there are any differences between this TR and the corresponding RTS and TRA, the RTS and TRA take precedence.

### Test report content

TR information	INF 54523319-11 v2

Test report content requirement.

1. First page is for those who doesn’t need to have the details but the result or don't need to read the whole report at this time, thus it needs to include what part is tested, what tests have been done, who did the tests and the result. For this reason, the first page shall at least include:

- Part and project name

- Supplier name

- Test lab

- Volvo Part no including version (maturity level)

- Tests performed

- Summary with conclusion.

2. For each separate test, there has to be enough facts for the reader to draw their own conclusions and, if desired: recreate the test in another lab. For this reason every test chapter (if several) shall at least include:

- Detailed test description including all applicable parameters, requirements, test sequence or test leg and how to perform the test. Including pictures of the setup. If the setup is too complex for pictures then an electric schematic is desirable.

- Description of test equipment including ID and calibration data.

- References to TR and/or STD and chapters. Initial measurements (Functional test) result, graphs, lists of

- measurements. Pictures

- Graphs, lists from measurements during the actual test. Both form DUT:s and equipment. Pictures

- Measurements between tests if the report is from a test sequence.

- Final measurements (Functional tests) result, graphs, lists of measurements. Pictures

- Results from opening and inspection. Pictures

- The suppliers detailed analysis of the result of the test

## Test preparations

### Initial and final status of component

HWC requirement	REQ 54523319-4 v2

ASIL Value:	Not Applicable

Each component shall be tested to perform its intended function as described by the manufacturer of the component, before and after testing.

All samples shall be visually inspected externally and opened for internal inspection after the test sequences have been completed.

- No defects which might affect the application, component or, if applicable, impression of the component quality is allowed.

- No unpleasant smell, mould or formation of fungus is allowed during or after the test.

- No cracks, corrosion, discoloration, spots, scratches, warping, fogging, fretting or other visible flaws or material degradations, or water intrusion onto/into PCB surfaces or voltage supplied structures are allowed.

- No rattling or noise is allowed.

- Labels on the component shall be attached to the component prior to C-sample testing. They shall be legible, have good adhesion and shall not smear after performed testing.

All samples shall be documented (e.g. by pictures and video clips) and a visual inspection report shall be part of the full set of test result reports provided to Volvo after test completion.

After inspection and test report creation, three (3) units shall be sent to Volvo (upon request). Two from climatic endurance, of which one has been subjected to damp-heat testing, one to high-humidity cyclic testing and one to the mechanical endurance test sequence.

#### Initial function test, normal climate

Verification method	VM 54523319-4 v2

Perform the test with the stated parameters at room temperature.

Pass criteria

FSC A

#### Initial function test, cold climate

Verification method	VM 54523319-5 v2

Perform the test with the stated parameters at T min.

Pass criteria

FSC A

#### Initial function test, hot climate

Verification method	VM 54523319-6 v2

Perform the test with the stated parameters at T max.

Pass criteria

FSC A

#### Final function test, normal climate

Verification method	VM 54523319-7 v2

Perform the test with the stated parameters at room temperature.

Pass criteria

FSC A

#### Final function test, cold climate

Verification method	VM 54523319-8 v2

Perform the test with the stated parameters at Tmin.

Pass criteria

FSC A

#### Final function test, hot climate

Verification method	VM 54523319-9 v2

Perform the test with the stated parameters at Tmax.

Pass criteria

FSC A

#### Post-test analysis

Verification method	VM 54523319-10 v2

All components shall be tested to ensure they are operating as designed, following all tests.  Any components that fail to perform as designed, shall be sent to the component manufacturer for analysis and failure report.

Pass criteria

A summary of each component’s condition shall be documented and reported to Volvo in the test reports.

### Monitoring

HWC requirement	REQ 54523319-5 v2

ASIL Value: Not Applicable

The output shall be continuously monitored during all tests and recorded for at least 100 cycles/20 seconds before and after any occurrences outside the specification, unless otherwise stated.

During testing in operating mode 2 or 3, enough parameters shall be monitored to ensure that the applicable FSC is fulfilled. Parameters and signals to be monitored shall be reviewed and agreed with Volvo.

#### Monitoring – Design review

Verification method	VM 54523319-11 v2

The stated parameters to monitor shall be agreed before any testing starts.

Pass criteria

The list of parameters shall be confirmed in the test description.

# Quality and Uptime

TR information	INF 54523319-12 v2

Refer to ‎1.9.1 for evidence of test results.

## Reliability/Durability

TR information	INF 54523319-13 v2

The minimum expected lifetime of the COTS components provided by Aurora is expressed in years and km.

During the expected lifetime of COTS components provided by Aurora, the component shall perform the intended functions under the stated conditions without failure and resist ageing.

### Reliability expectancy

HWC requirement	REQ 54523319-6 v2

ASIL Value:	Safety Related No ASIL

The COTS components shall demonstrate a minimum reliability of 90% with a confidence level of 90% for a period of 20000 system operating hours when operated in conditions relevant to the class 8 truck market for autonomous vehicles within North America.

#### Reliability expectancy - Design review

Verification method	VM 54523319-12 v2

Volvo and supplier shall review the design review for reliability expectancy.

Pass criteria

Minimum reliability value with a confidence level according to requirement demonstrated by calculation.

#### Reliability expectancy – Test

Verification method	VM 54523319-13 v2

Required reliability shall be demonstrated by testing sufficient samples until failure on highest critical failure modes/mechanisms identified in D/S FMEA. Alternatively a ‘success-test’ may be proposed to demonstrate required reliability and confidence level using existing test legs presented in the TR. Design of such tests, acceleration factors and sample size shall be agreed with Volvo. Time/cycles to failure in terms of normal usage shall be provided to Volvo for Reliability evaluation.

Pass criteria

Minimum reliability value with a confidence level according to requirement demonstrated by calculation.

### Vibration

HWC requirement	REQ 54523319-7 v3

ASIL Value:	Safety Related No ASIL

During the lifetime of the vehicle, the component shall in its assembled position endure all vibration levels generated by the truck and the operating environment.

NOTE: The following Random and Sinus Vibration tables are only for preliminary design information. Historical physical vehicle measurements or existing profiles in the form of component-specific or generic EEG are initially used to derive vibration profiles for the intended purposes of design guidance only. Vibration profiles are subject to change based on subsequent SVAT vehicle measurements that may reflect changes to the vehicle specification and/or component installation structure. Final test values might be higher or lower than initially provided and will supersede initial values. Components are subsequently required to meet requirements based on these final values.

#### Random vibration & temperature cycling – Long duration - Test

Verification method	VM 54523319-14 v3

Perform the test according to ISO 16750-3 with all parameters according to the standard but replace the duration and acceleration levels with the following values.

The following table shows the COTS list components which are subject to Random Vibration, Cab-Mounted Components:

Random vibration test shall be carried out with all connectors in place (connector integrity is part of the test), including a representative wire harness and fixations. The fastening and positioning of the test objects shall be equivalent to that in the actual vehicle.

Note: The test shall be performed with simultaneous vibration and temperature cycling (according to ISO 16750-4 5.3.1, with the exception below).

Operating mode 3.2 during ramping from Tmin to Tmax according to the figure below.

Operating mode 1.2 where no operation is required.

Pass criteria

FSC A and no cracks or breakage is allowed.

#### Random vibration - Short duration, 1-axis – Test

Verification method	VM 54523319-15 v3

Perform the test according to ISO 16750-3 with all parameters according to the standard but replace the duration and acceleration levels with the following values.

The following table shows the COTS list components which are subject to Random Vibration, Cab-Mounted Components:

Expose the component to random vibrations in the worst-case direction (Determined by the supplier) at room temperature and operating mode 3.2 for one hour in critical direction.

Pass criteria

FSC A and no cracks or breakage is allowed.

#### Random vibration – Short duration, 3-axis – Test

Verification method	VM 54523319-15 v3

NOTE: This test is optional.

Perform the test according to ISO 16750-3 with all parameters according to the standard but replace the duration and acceleration levels with the following values.

Expose the component to random vibrations for 1 hour per axis at room temperature and operating mode 3.2.

Pass criteria

FSC A and no cracks or breakage is allowed.

#### Sinus vibration & temperature cycling – Test

Verification method	VM 54523319-16 v3

Perform the test according to ISO 16750-3 with all parameters according to the standard but replace the duration and acceleration levels with the following values.

The following table shows the COTS list components which are subject to Random Vibration, Cab-Mounted Components:

Sinus vibration testing shall be carried out with all connectors in place (connector integrity is part of the test), including a representative wire harness and fixations.

The fastening and positioning of the test objects shall be equivalent to that in the actual vehicle.

Note: The test shall be performed with simultaneous vibration and temperature cycling (according to ISO 16750-4 5.3.1, with the exception below).

Operating mode 3.2 ramping from Tmin to Tmax according to figure in Verification method	VM 54523319-14 v2 Random vibration & temperature cycling – Test.

Operating mode 1.2 where no operation is required.

Pass criteria

FSC A and no cracks or breakage is allowed.

#### Mechanical shock – Test

Verification method	VM 54523319-17 v2

- Chassis-mounted components: Test according to ISO 16750-3 test 4.2.2.2.

- Cab-mounted components: Test using a modified ISO 19453 specification with the following parameters: 20g, 6mSec, half-sine; ± direction on each of 3 axes; 100 repetition per direction.

Mechanical shock testing shall be carried out with all connectors in place.

(Connector integrity is part of the test) including a representative wire harness and fixations.

The fastening and positioning of the test objects shall be equivalent to that in the actual vehicle.

Pass criteria

FSC A and no cracks or breakage is allowed.

#### Multi-axial rig / shake – Test

Verification method	VM 54523319-18 v3

The component should be installed on the multi-axial test rig (shake test …) in the same way as in/on the vehicle.

The load in/on the component shall be adapted to be as similar to actual customer usage as possible.

The rig will be subjected to road- and engine-related vibration that corresponds with Volvo test tracks.

NOTE: This is a Volvo-responsible test.

Pass criteria

FSC A

The component shall not move permanently out of its position during the test.

No defects in the shape or function of the component are allowed after the test has been performed.

#### AET – Test

Verification method	VM 54523319-19 v3

The components should be mounted on a test vehicle and subjected to the Accelerated Endurance Test (AET) at a Volvo test track.

If applicable, the component shall be loaded with weight that is similar to actual customer usage.

NOTE: This is a Volvo-responsible test.

Pass criteria

FSC A

No defects in the shape or function of the component are allowed after the test has been performed.

No cracks that lead to loss of structural integrity are allowed on the component after the test has been performed.

The component shall not squeak or rattle during the test.

### Dust intrusion

HWC requirement	REQ 54523319-8 v2

ASIL Value:	Safety Related No ASIL

The components and electrical connectors that make up the COTS components shall be resistant to intrusion of dust.

NOTE: The requirement is for the stand-alone Aurora component without Volvo-supplied covers.

#### Degrees of protection (1st IP code) – Test

Verification method	VM 54523319-20 v2

- Exterior-Mounted Components: Test according to ISO 20653:2013, IP first code number= IP6Kx.

- Interior-Mounted Components: N/A

Pass criteria

According to stated IP code.

#### Dust – Test

Verification method	VM 54523319-21 v2

Perform the test according to ISO 16750-4, chapter 5.10 Dust test.

Use the recommended parameters in ISO 16750-4.

Test dust shall consist of the below mixture (as suggested in the standard):

– 50 % by weight limestone (with clay and sand), i.e. “unbaked Portland cement”, and

– 50 % fly ash with the following grain size distribution (in accordance with IEC 60068-2-68):

– 33 % by weight ≤ 32 μm;

– 67 % by weight > 32 μm, but ≤ 250 μm.

Number of cycles 20

The functions shall be activated 10 times per cycle

1/3 of the components shall be brushed off after the test before functional testing

1/3 of the components shall be cleaned with compressed air before functional testing

1/3 of the components shall be tested without cleaning before functional testing.

Pass criteria

No intrusion of dust that can degrade the functional, electrical, mechanical or cooling properties is allowed.

### Moisture intrusion

HWC requirement	REQ 54523319-9 v3

ASIL Value:	Safety Related No ASIL

The components and electrical connectors that make up the COTS components shall be resistant to intrusion of moisture.

NOTE: The requirement is for the stand-alone Aurora component without Volvo-supplied covers.

The following table shows the COTS list components which are subject to Moisture Intrusion.

#### Permeability and operational – Test

Verification method	VM 54523319-22 v2

Perform a sealed temperature/humidity cyclic test in accordance with Volvo STD 515-0004.

Tmedium = +5 ºC

Exposure time: “Stabilization Time” + 10 min (measured in “Stabilization time test” (VM 54523319-30 v1), defined in the “Climate cycling” requirement in the TR).

Use either a plastic container or plastic bags for the test. If plastic bags are used:

- Seal the bags with a bag sealer (e.g. Quick-Seal Cut Model 300C)

- Ensure that all connectors and their wiring harness are connected to the component.

- Prevent moisture intrusion if no connector is attached to the other end of the wiring harness.

- Ensure that both ends of the wiring harness are sealed.

- The specification above requires an opening and inspection of the component after performed test.

- Do not open and inspect the component after performed permeability test.

Operating mode 1.2

Operational test

After testing in accordance with the standard specified above, perform an operational test.

Preparation:

Open the plastic container/plastic bags after (within 8 h) the testing specified above and place them immediately in a new plastic container/plastic bag and seal it.

Ensure that the components can be operated, i.e. that the wiring harness can be connected to power and/or controls.

Test duration: 10 days

Climate chamber temperature: 5 ºC

Operating mode 3.2, continuous monitoring

Functional test, normal climate to be performed within 2 h after performed combined permeability & operational test, test duration 24 h.

Pass criteria

FSC A during the operational test. No intrusion of moisture/water is allowed.

### Water intrusion

HWC requirement	REQ 54523319-10 v3

ASIL Value:	Safety Related No ASIL

The components and electrical connectors that make up the COTS components shall be resistant to intrusion of moisture.

NOTE: The requirement is for the stand-alone Aurora component without Volvo-supplied covers.

The following table shows the COTS list components which are subject to Water Intrusion.

#### Ice water shock, splash – Test

Verification method	VM 54523319-23 v2

Test according to ISO 16750-4 §5.4

Pass criteria

FSC A and no intrusion of water into the component.

#### Ice water shock, Submersion – Test

Verification method	VM 54523319-24 v2

Test according to ISO 16750-4 §5.4

Pass criteria

FSC A and no intrusion of water into the component.

#### Degree of protection (2nd IP code) – Test

Verification method	VM 54523319-25 v2

- Exterior-Mounted Components: Test according to ISO 20653, IP second code number= IPxK9K.

- Interior-Mounted Components: N/A

Pass criteria

FSC A and no intrusion of water into the component.

#### Salt spray – Test

Verification method	VM 54523319-26 v2

Perform ISO 16750-4 5.5.2 leakage and functional test.

Exception from ISO 16750

Operating mode 3.2 (during test duration)

Pass criteria

FSC A and no intrusion of water into the component. No corrosion is allowed inside the component.

## Atmospheric environment lifetime

TR information	INF 54523319-14 v2

Since this component is powered by the ADM (Aurora Driving Module) system and not the vehicle power bus, the Usmin, Usmax, Usnom are defined by Aurora for the tests. The value for these voltages shall be noted in the test report.

### Operating temperature

HWC requirement	REQ 54523319-11 v2

If no specific temperature profile is communicated by Volvo, the component shall be fully functional and meet all requirements in the range of –Tmin < T < Tmax for all possible temperature profiles during the component and vehicle life cycle

Tmin -40°C

Tmax 85 ºC

#### Cold temperature wake-up – Test

Verification method	VM 54523319-27 v2

Perform ISO 16750-4 § 5.1.1.1 Low temperature test, Storage test (off) for 24 h, but with operating mode 1.2. After 24 h still at Tmin, go to operating mode 3.2 with Usmin supply voltage and monitor electrical functionality including the on transition. Cycle the power ON (100 s) / OFF (20 s) for 1 h and monitor that each ON period starts and functions correctly until power off.

Pass criteria

FSC A while powered for each ON cycle.

#### High temperature, operation – Test

Verification method	VM 54523319-28 v2

Long-term heat resistance for electrical components

Test duration: 500h

Test according to ISO 16750-4 5.1.2 High temperature test, Operation test

Operating mode 3.2, continuous monitoring

The test operating voltage shall be Usnom for 80 %,

Usmin for 10 % and Usmax for 10 % of the test time.

Pass criteria

FSC A. Geometry gaps and flushes according to specification.

### Sunshine

TR information	INF 54523319-15 v2

Sunlight is part of the electromagnetic radiation generated by the Sun, in particular infrared, visible, and ultraviolet light. Sunshine is a combination of bright light and radiant heat.

HWC requirement	REQ 54523319-12 v3

The following table shows the COTS list of components which are subject to sunshine. The table identifies which surfaces of these components shall be resistant to degradation due to sunlight and sunshine, combined with relevant atmospheric conditions such as high temperatures, humidity and wetting.

| Aurora COTS Component | Environmental stress requirement needed? |
| --- | --- |
| Taoglass Antenna Combo | Yes: functional and cosmetic |
| Ublox GNSS Antenna | Yes: functional and cosmetic |
| Hänsch Loudspeaker | Yes: functional and cosmetic |
| Hosiden Microphone | No: not exposed to direct sunlight |

#### Light exposure (exterior) – Test (If applicable)

TR information	INF 54523319-16 v3

Accelerated ageing for exterior plastics substrates and painted surfaces, by artificial light, in combination with raised temperature and intermittent wetting

Verification method	VM 54523319-29 v2

Accelerated ageing test should apply to any plastic parts which are exposed to light and controlled by the supplier.

According to STD 1027,337 Light exposure – Accelerated ageing of exterior material – Test Method

Temperature according to STD

Exposure time: 1500 h for surfaces which represent true exposure in the vertical position and 2000 h for surfaces which represent true exposure in the horizontal position.

Pass criteria

The system performance should remain unaffected. In the event of any performance degradation, the system should have the capability to detect it. Additionally, Volvo shall be informed of the test results.

### Climate cycling

HWC requirement	REQ 54523319-13 v2

The component shall be able to withstand and maintain functionality in varying temperatures, e.g. when the components are used in changing ambient temperatures. (Tmax–Tmin).

#### Stabilization time – Test

Verification method	VM 54523319-30 v2

Before starting any climate cycling test, the stabilization time (ts) of the component shall be measured.

Stabilization occurs when the temperature difference (ΔT) between the test specimen and the test medium is within Tmax – 3°C or Tmin +3 °C. This measurement shall be carried out by applying a thermocoupler to the part of the component with highest thermal mass.

Take into account that the stabilization time will also depend on the chamber and the number of components being tested.

Pass criteria

Stabilization time for the largest thermal mass in the component.

#### Rapid change of temperature (thermal shock) – Test

Verification method	VM 54523319-31 v2

Number of cycles, Rapid change of temperature I (see Test overview, leg 1) 800.

Number of cycles, Rapid change of temperature II (see Test overview, leg 2) 100.

Use test method according to IEC 60068-2-14 Test Na Rapid change of temperature with prescribed time of transfer.

Operating mode: 1.2 connected to wire harness but not powered.

Dwell time: see Stabilization Time.

Adjust Tmax to compensate for internal heating. Example: If a component is specified for 85 °C and internal heating is 10 °C, then Tmax=95 °C.

TA=Low temperature in °C

TB=High temperature in °C

T, temperature in °C

t, time

t1=10min, Duration of exposure after temperature stabilization (See chapter Verification method	VM 54523319-30 v2)

Pass criteria

FSC A

#### Temperature cycling – Test

Verification method	VM 54523319-32 v2

Use test method IEC 60068-2-14 Test Nb. Change of temperature with specified rate of change

Operating modes in accordance with TR /Test conditions chapter (function definition only for operating mode 3.2).

3.1 if sleep mode is applicable / 1.2 if sleep mode is not applicable, during the negative thermal transition and the low-temperature phase.

3.1 if sleep mode is applicable / 1.2 if sleep mode is not applicable and 3.2, respectively, during the positive thermal transition and the high-temperature phase. When Tmin + 3 C is reached, begin cycling ON (Operating mode 3.2) 100 s and OFF (Operating mode 3.1/1.2) 20s until the next negative thermal transition begins.

Operating mode 3.2

Monitor continuously. In operating mode 3.2, the test voltage shall be Usnom for 80 %, Usmin for 10 %, and Usmax for 10 % of the completed cycles.

Use the following parameters:

Temperature range: Tmin to Tmax (denoted TA and TB in IEC 60068-2-14)

Transition time: 2–15 °C/min,

Duration of exposure: see Stabilization Time (Verification method	VM 54523319-30 v2)

Number of cycles: 100 (minimum 100 cycles)

TA=Low temperature in °C

TB=High temperature in °C

T, temperature in °C

t, time

t1=10min, Duration of exposure after temperature stabilization (See chapter Verification method	VM 54523319-30 v2)

Pass criteria

FSC A

#### Temperature step – Test

Verification method	VM 54523319-33 v2

Test according to ISO 16750-4 § 5.2 Temperature steps test

Operating mode 3.2 at each temperature step

Exception: Follow ISO 16750-4 § 5.2, but increase the temperature steps as follows:

After reaching T max, decrease the temperature in steps according to the standard, to +20 °C.

Pass criteria

FSC A

### Process oven temperature

HWC requirement	REQ 54523319-14 v3

The following table shows the COTS list components which are subject to Process oven temperature.

For any components expected to remain mounted in place on the vehicle during any aftermarket paint booth / paint curing processes, then the component shall be able to withstand the painting process and oven temperatures (Tmin 100 °C) while not operating.

For any component(s) unable to withstand the painting process oven requirement above, Aurora shall notify Volvo so that operations can be instructed to remove the component(s) prior to the painting / curing process.

For components requesting exemption, the component supplier shall acknowledge during the RTS and also state during the PHR (Part Handling Review) process.

#### Storage temp aftermarket paintshop – Test

Verification method	VM 54523319-34 v2

Place the component in a hot air oven at 100 °Celsius for 2 hours.

For components requesting exemption, the component supplier shall acknowledge during the RTS.

Pass criteria

No cracks or deformations and no degradation of functionality.

### Moisture

HWC requirement	REQ 54523319-15 v2

The components and electrical connectors that make up the COTS components shall be resistant to moisture, both in form and function, from very dry up to 95 +/–5 % relative humidity.

No unpleasant smell, mould or formation of fungus is allowed during normal handling, transport, storage or mounting.

NOTE: The requirement is for the stand-alone Aurora component without Volvo-supplied covers.

#### Composite temperature/Humidity cycling – Test

Verification method	VM 54523319-35 v2

Perform the test according to IEC 60068-2-38 Z/AD, except for the requirements on functional cycling and monitoring. Instead, apply the requirements below.

Operating mode: Cycle between ON (Operating mode 3.2) for 1 h and Sleep mode (op. mode 2.1 or OFF if the component has no Sleep mode) for 1 h, starting with the first hour OFF.

Between hour 17 and 21, the unit shall be OFF/in Sleep mode. Supply voltage: Usnom.

During the ON mode, continuous monitoring is required. During the Sleep/OFF mode, continuous parasitic current is monitored and recorded over the 10-day test period to detect any malfunctions during the test.

Pass criteria

FSC A. No increase in parasitic current greater than 50 % is allowed.

#### Damp heat, steady state – Test

Verification method	VM 54523319-36 v2

Perform the test according to IEC 60068-2-78, Damp heat, steady state, but with the following parameters:

Temperature: +65 ± 3 ºC

Relative Humidity: 93 ± 3 % R.H.

Duration: 10 days.

Operating mode: Cycle between ON (op. mode 3.2) for 1 h and Sleep mode (op. mode 2.1 or OFF if the component has no Sleep mode) for 1 h. To avoid local drying of the component, the heating should be minimized and other ON/OFF cycling might be necessary. Supply voltage: Usnom. During the ON mode, continuous monitoring is required. During the Sleep/OFF mode, continuous parasitic current is monitored and recorded over the 10-day test period to detect any malfunctions during the test.

Pass criteria

FSC A. No increase in parasitic current greater than 50 % is allowed. No signs of electro-migration or dendritic growth at final inspection.

### Air pollution

HWC requirement	REQ 54523319-16 v2

The component’s functionality shall not be affected by the surrounding air, both clean and polluted.

The operation, functionality and surface of the component shall not be adversely affected by air pollution when used in the vehicle.

#### Corrosion test with flow of mixed gases – Test

Verification method	VM 54523319-37 v2

Expose the component to a mixture of three gases with concentrations according to table below in a MFG (Mixed Flowing Gas) climatic chamber

| Gas | Concentration [ppb] |
| --- | --- |
| H2S | 2000 |
| NO2 | 200 |
| SO2 | 200 |

Temperature: +4 0°C ± 3 °C

Test duration: 504 h

Relative humidity: 75 ± 3 %

Operating modes: 2.1 (OFF) and 3.2 (ON),

Test cycle: 1 hour ON, 1 hour OFF

Check the functionality of the test objects after one and two weeks (operating mode 3.2).

Pass criteria

FSC A

Note: The device may show signs of corrosion.

The following items shall be monitored and shall not fail or cause failures in the test:

– Copper creep shall not have short circuited any electrical nets

– All chip resistors shall be functional with correct resistance values

– All LEDs, including any with silicone-optical protections, shall function normally. (A bare-eye inspection is sufficient.)

## Quality impression

TR information	INF 54523319-17 v2

Quality Impression (QI) is the perception of the product quality with engine off; at 0 km. Quality impression is an important aspect and this component shall support the overall quality impression as specified by Volvo.

### Surface finish

TR information	INF 54523319-18 v2

Not applicable.

### QI Solidity

TR information	INF 54523319-19 v2

Solidity is the perceived quality of attachment, material rigidity, sound and deterioration

HWC requirement	REQ 54523319-17 v2
General

• All materials must be fixated in a sufficient and appropriate number of points to fulfill flex and attachment requirements

• There shall be no perceivable freeplay when knocking/tapping on assembled parts

• No parts or surfaces shall be perceived as being weak or too flexible in any direction.

• All moveable/adjustable parts shall deliver a solid perception and should only be moveable in the, for the function, intended direction (e.g. lids, doors, drawers, handles)

• There shall be no perceived noise such as creaking, squeaking or rattling when operating part(s) including pushing/pulling and knocking on part

#### QI Solidity – Evaluation

Verification method	VM 54523319-38 v3

Evaluation to occur on B- and C-builds using representative parts from Aurora. Volvo Product Development Group to provide input as well as design review.

Pass criteria

The fulfilment of the requirements is verified by a review of the component design with the concerned stakeholders.

### Corrosion

HWC requirement	REQ 54523319-18 v2

The component shall not adversely affect the quality impression of the vehicle or impede functionality.

#### Corrosion test on complete vehicle – Test

Verification method	VM 54523319-39 v2

Test according to SAE J2721 complete vehicle corrosion test.

Pass criteria

To be defined

### Scratch resistance

HWC requirement	REQ 54523319-19 v3

The component surface shall be resistant to scratching, both during manufacturing and over the vehicle’s expected lifetime.

NOTE: Scratch resistance tests only apply to components which can have functions affected by scratches.

#### Scratch resistance by Erichsen pen – Test

TR information	INF 54523319-20 v3

The purpose of this verification method is to assess the appearance of materials with small scratches that occur during manufacturing and over the vehicle’s expected lifetime.
NOTE: Scratch resistance tests only apply to components which can have functions affected by scratches.

Verification method	VM 54523319-40 v3

According to STD 423-0030 Scratch resistance, Erichsen pen - Organic materials

Method A, needle 2, Spring 2 (ERICHSEN PEN)

Force load: 3 N minimum

Pass criteria

The system performance should remain unaffected. In the event of any performance degradation, the system should have the capability to detect it. Additionally, Volvo shall be informed of the test results.

NOTE: Scratch resistance tests only apply to components which can have functions affected by scratches.

#### Scratch resistance (abrasive paper) – Test

TR information	INF 54523319-21 v3

The purpose is to assess the appearance of materials with hard scratches that occur during manufacturing and over the vehicle’s expected lifetime.

NOTE: Scratch resistance tests only apply to components which can have functions affected by scratches.

Verification method	VM 54523319-41 v3

Test according to STD 423-0040 Scratch resistance using abrasive paper - Organic materials.

NOTE: Scratch resistance tests only apply to components which can have functions affected by scratches.

Pass criteria

According to STD 423-0040 Scratch resistance using abrasive paper - Organic materials

The grey-scale assessment shall be ≥ 4 according to ISO 105-A03 Scale – staining (alt. Volvo STD 1026,8202 Grey scale – staining).

### Chemicals

HWC requirement	REQ 54523319-20 v2

The operation, function and surface of the component shall not be adversely affected by accidental liquid splash or spray, etc. of any type of chemical agents, detergents or liquids used in the vehicle over the expected lifetime.

The surface shall not be stained due to contact with other adjacent materials such as varnishes, plastics, rubber, etc.

#### Resistance to chemical loads – Test

Verification method	VM 54523319-42 v3

Test according to Table 1 in ISO 16750-5 with mounting location for each device as indicated in the table below:

Operating mode: 1.1.

Chemical exposure and Application method: According to ISO 16750-5, with modifications according to table below:

Pass criteria

The minimum functional status shall be class A in accordance with ISO 16750-1.

The surfaces and material shall not be degraded.

Any marking and labelling shall remain visible and legible.

#### Ageing in fluids (plastic/rubber) – Test (If applicable)

Verification method	VM 54523319-43 v3

Applicable only to components which are in contact with the fluids mentioned.

Test according to ISO 1817 Rubber, vulcanized – Determination of the effect of liquids.

Temperature: 85 °C

Pass criteria

No cracks or dimensions outside of tolerances are allowed.

### Material and composite materials properties

HWC requirement	REQ 54523319-21 v2

The (composite/plastic/metal) material properties shall contribute to fulfilment of all component requirements, over the component’s expected lifetime.

The plastic material properties shall fulfil the requirements in STD 411-0001 Plastics.

The requirement is considered for Aurora-supplied component without the Volvo-supplied covers.

#### Material properties – Documentation

Verification method	VM 54523319-44 v2

Documentation (e.g. supplier’s Material Data Sheet) shall certify that the material’s properties are in accordance with the specified requirements.

Pass Criteria

Documented/stated compliance with the requirements.

## Maintainability

TR information	INF 54523319-22 v2

Maintainability is the ease of performing routine maintenance such as Daily maintenance ( i.e. checking the oil level), Preventive maintenance (i.e changing the oil and filter), Corrective maintenance (repairs), and Diagnosis.

If Aurora components have any effect on volvo components, they need to abide by the following maintainablity reqs and should coordinate with any other project workgroups focusing on this area, including required periodic maintenance, maintenance intervals, etc.

### Design for maintainability

HWC requirement	REQ 54523319-22 v2

Maintenance operations

- The use of glue to assemble parts shall be avoided for preventive/corrective maintenance of parts.

- Replacement of components shall not require a recalibration. If a recalibration cannot be avoided, it shall as far as possible be automatic and not require any special tools (e.g. scales) or a diagnostic application (such as TechTool). A software download shall never require a manual recalibration.

- Uptime-critical components (that cause breakdown or unplanned maintenance) shall be easy to access (removal of bolts, connectors, joints, etc.) for diagnostic operations (e.g. manual measurements).

- Removal of unrelated components shall not be required for preventive and corrective maintenance.

- The need to open panels, hatches or tilt the cab in order to perform daily maintenance shall be avoided.

- The fault tracing time shall be considered in component design in complex installations.

Quick connectors

- If no Standard parts are used for connection, quick connectors shall be provided for all pneumatic and fuel routings and easy-disconnect connectors for wiring harnesses.

Tools

- The component design shall be optimized to limit the number of standard workshop tools needed to perform maintenance operations, independent of installation or application.

- The use of special tools shall not be required.

- No standard tools shall be required to disconnect quick connectors.

Fluid draining/filling/checks

- The filling and draining rate of fluids shall be at least 10 litres/min for proper upkeep and operation.

- It shall be possible to check fluid levels without dismantling the ingoing parts of the component (e.g. removal of filler plug for oil-level check in drive axle is acceptable).

Lifting aid on the component

- Components heavier than 5 kg shall be provided with a lifting aid, such as a hook, lifting eye, keyhole.

Replacement durability

- Replacement of components shall not require destructive removal of related components (e.g. a removal of the fork from the shaft in a differential lock shall not require drilling out the lock pin).

- Components with snap-fits, for example clips, shall have a replacement durability of at least 25 cycles.

#### Design for maintainability – Design review

Verification method	VM 54523319-45 v2

Compliance with design for maintainability requirements is verified through a review of the concept solution documentation in an early development phase.

Pass criteria

Maintenance operation time, convenience evaluation and service contract cost

Documented/stated compliance with the specified requirements.

# Safety

TR information	INF 54523319-23 v2

The areas considered for the safety tests are collision safety, operator safety and fire protection. Aurora shall be responsible for supporting Volvo in all of the required simulations per each unique installation, and shall provide manufacturer datasheets / evidence of compliance to automotive safety standards.

# Driver (operating) environment

TR information	INF 54523319-24 v2

Refer to ‎1.9.1 for evidence of test results.

Aurora shall work with Volvo to ensure that none of the COTS components (nor installations) contribute to the following:

- Noise

  - Squeak and rattle

  - Wind noise

  - Noise generated by electrical devices

# Environment – Materials and environmental impact

TR information	INF 54523319-25 v2

Refer to ‎1.9.1 for evidence of test results.

The Volvo Group’s goal is to be ranked as a leader in terms of Environmental Care among the world’s top producers of transport-related products, equipment and systems. The Volvo Group’s Environmental Care programmes shall be characterized by a holistic view, continuous improvement, technical development and resource efficiency. These means shall give the Volvo Group a competitive advantage and contribute to sustainable development.

Focus areas are Choice of material, Resource efficiency, Reduction of emissions, Waste management.

Each new component shall strive for an environmental improvement in comparison with its predecessor.

## Design for recycling

HWC requirement	REQ 54523319-23 v2

General

- All materials and components shall be suitable for recycling in the end-of-life phase, and the risk of exposure to toxic materials shall be minimized and declared.

- Special attention shall be paid to component accessibility, ease of extraction at end-of-life, dismantling, and the existence of a recycling process.

- Dismantling information shall be provided.

Attachment components (Nuts, screws, clips, etc.) shall be:

- standardized and the number of attachment components shall be reduced to a minimum.

- “rapid assembly systems”, such as clips, if possible.

Reservoirs and fluid/gas systems (for fluids and gases) shall:

- be designed with a single low point. (At end-of-life, fluids will be extracted through suction or by gravity.)

- be accessible (windscreen washer, brake and cooling fluids, etc.).

- have an emptying plug, i.e. fuel, windscreen washer fluid, brake fluid, cooling fluid, oils (engine, gearbox, drive axle, power steering, shock absorbers, etc.), air conditioner fluid, etc.

Metal inserts – If metal inserts are used in plastic components, limit their size.

Disassembly – All components shall be easy to disassemble/dismantle, including:

- Components that need to be treated in separate processes (e.g. oil filters, batteries, etc.)

- No-n-organic, non-metallic materials, e.g. windows, especially glued windows

- Batteries. If the component is equipped with batteries, the recommendations in the battery directive 2006/66/EC—2008/103/EC Directive on batteries and accumulators, shall be followed.

- If pyrotechnic functions (e.g. pre-tensioners and airbags) cannot be dismantled, design a simple, efficient neutralization system.

### Design for recycling – Design review (If applicable)

Applicable only to components which are in contact with the fluids mentioned.

Verification method	VM 54523319-46 v3

The fulfilment of the requirements shall be verified by a review of the component design with the concerned stakeholders.

The design for recycling and minimization of environmental impact shall be approved by Volvo in a design review before B-release.

Pass criteria

Documented/stated compliance with requirements.

## Material selection

HWC requirement	REQ 54523319-24 v2

- Materials shall be selected to minimize the environmental impact regarding hazardous substances, recyclability, waste management, and material emissions.

- Take into consideration how the component and material can be refurbished, remanufactured to make the useful life of the component and material longer and in the end recycled.

- Solvent-borne surface treatment shall not be used in the cab interior. This requirement does not apply to PCB coating and treatment of electrical cables.

Polymer materials

- Whenever possible, use a single family of polymer materials, for instance PP family or ABS family.

- All components in polymer materials shall be evaluated and considered for design using recycled material, when the required properties and characteristics are achieved. For all non-visible components, it is recommended to use recycled polymers. Components made from recycled material shall be correctly labelled.

- To minimize the production waste of plastic, recycled plastic materials can be used as long as the properties of the component are not compromised, and the parts are correctly labelled.

- PVC shall only be used without hazardous additives.

### Material selection – Design review

Verification method	VM 54523319-47 v2

The fulfilment of the requirements is verified by a review of the component design with the concerned stakeholders.

The material selection shall be approved by Volvo in a design review before C-release / freeze of tool design.

To support the review use Volvo Group Eco-design tools.

Pass criteria

Documented/stated compliance with requirements.

## Substances that shall not be present in product

HWC requirement	REQ 54523319-25 v2

The component shall fulfil the requirements in STD 100-0005 “Volvo´s red list” – Chemical substances which shall be declared and substances that must not be present in Volvo Group products placed on the market

- If applicable, components located in the cab interior shall be free from Brominated Flame Retardants (BFR).

- If a chemical substance needs to be labelled, applicable labelling requirements shall be followed.

- If a biocide is used, the active substance shall be approved for the product type according to the EU Biocide directive. If a part/product is treated with a biocide, it shall be labelled accordingly. This requirement also applies when no biocide is left in the product.

- If transport protection for example corrosion protection oils are used or the component contain chemical products secure that STD 100-0002 and STD 100-0003 are fulfilled.

### Substances that shall not be present in product – Reporting

TR information	INF 54523319-26 v2

Supplier Substances reporting is mandatory in Technical P-release (frozen reporting) and reporting is requested from C-release. An accepted MDS is the condition for PPAP acceptance. The detailed process for PPAP acceptance will be communicated by the Purchaser on the Volvo Group Supplier Portal.

When point 4. above is applicable the chemical content of the chemical product needs to be reported.

Verification method	VM 54523319-48 v2

Reporting according to STD 100-0006 MDS (Material Data Sheet) reporting of substances and material composition to IMDS (International Material DatabaSe) by the supplier.

Reporting by the supplier of all process chemicals, within the Volvo Group, into Flecs (Volvo France), MoTiV, or other, equivalent system used within the Volvo Group.

Pass criteria

Documented/stated compliance with requirements.

## Emissions and odours

TR information	INF 54523319-27 v3

Applicable only for interior components; Not applicable for the microphone (due to physical size & makeup).

In the event that any larger COTS components are used in the cabin interior, they will be subject to tests to check for emissions of volatile organic compounds such as formaldehyde, etc.

# Driving performance

# Transport effectiveness

## Component weight

TR information	INF 54523319-28 v2

The truck GCW (Gross Combination Weight), GVV (Gross Vehicle Weight), Axle Weight and dimensions are regulated by law through a Certificate of Conformity.

The weight also has a strong influence on the features Load Capacity, Fuel Consumption, and Environmental Impact.

Legislation and features put pressure on developers to optimize the weight of produced trucks in order to increase the payload for the truck and make it help customers to increase their income.

HWC requirement	REQ 54523319-26 v2

The weight shall be reduced as much as possible, and kept under control.

The weight should be documented and any time it changes, Volvo shall be notified.

### Component weight – Documentation

Verification method	VM 54523319-49 v2

Verification by documentation of estimated weight or weighed component, in Volvo’s system (KOLA) at registration of a new part number.

Updates of accuracy at the (A, B, C, P) releases, for example from CAD calculations or weighed prototype parts.

At PPAP, an average of 5 weighed components shall be documented in Volvo’s systems.

Pass criteria

Documented/stated compliance with requirements.

# Vehicle regulation and Regulative conditions

## Vehicle regulations and certification

HWC requirement	REQ 54523319-27 v2

The component shall comply with all relevant regulations, including applicable installation regulations (that will affect the component) on the markets where it will be sold, according to the list below

| Requirement Area | Market | Regulation / Certification reference (including series supplement and year when applicable) / Standards connected to legal requirement | Is a certificate or test report required | Required type approval marking (on component or drawing) |
| --- | --- | --- | --- | --- |

Aurora shall

- notify Volvo if any of the COTS components are subject to  any Federal, State, or local regulations or certifications

- provide documentation to Volvo prior to C-release that the COTS components meet required regulations or certifications

### Vehicle regulations and Certification (xxx) – Design review

Verification method	VM 54523319-50 v2

To be confirmed in documentation.

Pass criteria

Documented/stated compliance with requirements.

# Electrical & Electronics

TR information	INF 54523319-29 v2

Refer to ‎1.9.1 for evidence of test results.

## E&E mechanical properties

### Silicone-free

HWC requirement	REQ 54523319-28 v3

The use of silicone as a cleaning agent, release agent, lubricant, paste, adhesive, inside dampers or any other non-cross-linked or uncured silicone where risk of leakage of volatile silicone to surroundings occur is not permitted.

In general, cross-linked silicone such as silicone rubber or cured silicone rubber shall not emit any low-molecular agents that can be harmful or have any functional impact on electrical appliances or in paint shop.

Any silicone materials used in the design of units with electrical functionality, including mechanical switches, micro switches and relays controlling inductive loads, shall be reviewed with Volvo. This includes silicone oil, silicone grease, and silicone rubber parts. The migration of silicone molecules on surfaces that can see electrical arcing will cause open-circuit failures over time.

#### Silicone- free – Design review

Verification method	VM 54523319-51 v2

Use of any material containing silicone shall be reviewed, discussed and documented.

Pass criteria

Documented/stated compliance with the specified requirements.

### Sulphur-free rubber

HWC requirement	REQ 54523319-29 v2

No sulphur-cured rubber material is allowed inside of the Component.

Peroxide-cured material or silicone-rubber materials shall be used.

This requirement also applies to rubber sealings in electrolytic capacitors.

#### Sulphur-free rubber – Design review

Verification method	VM 54523319-52 v2

The sulphur content shall be discussed and approved before B-release.

Pass criteria

Documented/stated compliance with the specified requirements.

### Lead-free

HWC requirement	REQ 54523319-30 v2

Electrical and Electronic Assemblies shall have a lead-free design.

The definition of a lead-free requirement is a quantity limit of 0,1 % by mass (1000 ppm) in homogeneous material for Lead (Pb).

Examples of electrical and electronic assembly materials:

• Solder used to connect electrical and/or electronic parts to a circuit board (substrate) or a lead frame (without a substrate)

• Coating on connector terminals and "pre-soldering" of wires and components prior to soldering, crimping etc. (See Annex II for pressfit / compliant pin exemption)

• Bare PCB surface finish for soldering preservation

• Protective coatings on the circuit board (Conformal coat materials)

• Shrink hoses for components on the circuit board.

#### Lead-free – Design review

Verification method	VM 54523319-53 v2

The lead-free content shall be identified during RTS.

Pass criteria

Documented/stated compliance with requirements.

### Connectors

HWC requirement	REQ 54523319-31 v3

Connectors used shall be from Volvo connector palette.

If the component is equipped with more than one connector of a certain type, all of the connectors shall be individually and mechanically coded to achieve "poka yoke". If possible, colour coding in combination with mechanical coding is also preferred.

The connectors that need to be in TR are the ones that Volvo has control over.

Either the wiring and/or connectors used must be automotive grade connectors/wiring or a risk analysis must be performed to evaluate the risks associated if no automotive grade components are used.

#### Connector – Design review

Verification method	VM 54523319-54 v3

The fulfilment of the requirements is verified by a review of the component design with the concerned stakeholders.

The requirement applies to the connectors that Volvo has control over.

Either the wiring and/or connectors used must be automotive grade connectors/wiring or a risk analysis must be performed to evaluate the risks associated if no automotive grade components are used.

### Electrical wiring

HWC requirement	REQ 54523319-32 v3

Applicable for components with wiring pigtails.

Aurora shall inform Volvo about the type, length and resistance of wires and cables.

The pigtail should be designed so that it can tolerate the weight of the component if it is held by the wires.

The following standards needs to be considered for the design.

Applicable for all wires

STD 525-0001 Wires - Dimensions, test methods and requirements

STD 525-0004 Cables - Dimensions, test methods and requirements

Either the wiring and/or connectors used must be automotive grade connectors/wiring or a risk analysis must be performed to evaluate the risks associated if no automotive grade components are used.

### Pin description

HWC requirement	REQ 54523319-33 v3

Required signals for different connectors are listed below. Pin assignment shall be done by supplier based on PCB design and layout. The PCB track lengths of differential signal pair between SoC outputs and connector terminals shall be the same.

The Pins that need to be in TR are the ones that Volvo has control over.

Either the wiring and/or connectors used must be automotive grade connectors/wiring or a risk analysis must be performed to evaluate the risks associated if no automotive grade components are used.

| Connector number | Pin number | Pin name | Function name |
| --- | --- | --- | --- |
| 1 |  | TBD | TBD |
| 1 |  | TBD | TBD |

| Connector number | Pin number | Pin name | Function name |
| --- | --- | --- | --- |
| 2 |  | TBD | TBD |
| 2 |  | TBD | TBD |

#### Pin description – Documentation

Verification method	VM 54523319-55 v3

Shall be verified in the component documentation.

Pass criteria

Shall be confirmed by all stakeholders.

Either the wiring and/or connectors used must be automotive grade connectors/wiring or a risk analysis must be performed to evaluate the risks associated if no automotive grade components are used.

### Printed Circuit Board (PCB) assembly

HWC requirement	REQ 54523319-34 v3

The following table shows the COTS list components which are subject to Printed Circuit Board (PCB) assembly and/or PCB Conformal Coating

The choice of PCB type used material qualifications and workmanship shall be according to the pass criteria of IPC J-STD-001 class 3 for safety-related components.

The types of fluxes used for the soldering processes shall be according to IPC J-STD-004.

Allowed flux classification according to J-STD-004 ROL0, ROL1, REL0 and REL1.

Use No Clean type. Do not clean the boards unless otherwise agreed on.

Do not use water-soluble fluxes

Acceptable substrate materials FR4-types, CEM3 (low-population boards), Polyimide, Ceramic.

Any shaping and cutting of the PCBs shall not damage the PCB, initiate cracks in the soldering or damage the components.

Bare PCB surface finish

Acceptable bareboard surface finishes on organic substrates (FR4, CEM3 etc.) are:

ENIG, HASL, ImSn, OSP.

Silver material surfaces ImAg SHALL not be used.

#### PCB assembly – Design review

Verification method	VM 54523319-56 v2

The PCB properties shall be identified during RTS.

Pass criteria

To be confirmed in documentation and process description.

### PCB Conformal Coating

HWC requirement	REQ 54523319-35 v2

All PCBs shall be protected by conformal coating or corresponding method which provides electrical insulation and sufficient resistance to corrosion. The supplier shall present the choice of method and type of lacquer at the start of the design phase. The supplier shall also secure the coating’s feasibility in the layout design. If the PCB is not completely protected by lacquer, this shall be approved by Volvo.

The thickness of conformal coating shall be as follows as the type specified. If other than the following types of conformal coating being used, this shall be approved by Volvo.

| Coat type | Short name | Curing remarks | Coating thickness, in mm, in accordance with IPC J-STD-001 |
| --- | --- | --- | --- |
| Acrylic | AR |  | 0,03–0,13 |
| Epoxi | ER |  | 0,03–0,13 |
| Silicone | SR |  | 0,05–0,21 |
| Urethane | UR | Do not use oxygen curing coating | 0,03–0,13 |
| Parylene | XY |  | 0,01–0,05 |

Solvent-free coatings shall be used for all materials. Do not use water as a solvent.

Coating Processes: Dip, Spray, Selective Spray, Vapor Deposit

Coating Material Selection Matrix

|  | AR | ER | SR | UR | XY |
| --- | --- | --- | --- | --- | --- |
| Operating temp –40 – 85 °C | OK | OK | OK | OK | OK |
| Operating temp –40 > +85 °C | NOK | OK | OK | NOK | OK |
| IP Code ≤5k3 | OK | OK | NOK | OK | OK |
| IP Code ≥5k4 | OK | OK | OK | OK | OK |

Note 1: For the operating temperature range above +85 °C, AR-and UR-conformal coating materials shall not be used due to the mechanical forces created during thermal cycling for parts where the coating applies external forces on solder joints (Minimelf, QFP, BGA, PLCC, etc.)

Note 2: Silicone-conformal coatings in combination with sulphur vapors from rubber or air pollution are very aggressive on silver-content parts like chip resistors and LED lead frames. Open-circuit faults can be created over time.

#### PCB conformal coating –Design review

Verification method	VM 54523319-57 v2

To be discussed and agreed before B-release

Pass criteria

To be confirmed in documentation.

## E&E Production requirements

### End Of Line functional – Test

HWC requirement	REQ 54523319-36 v2

Each delivery unit must pass the functional test after assembly. Volvo and the supplier shall jointly specify the measurements that will be performed. Also, the dimensions and requirements shall be checked and agreed with Volvo. The test environment shall be supplied by the supplier. Possible test software shall be developed by the supplier. The documentation of all production line tests shall be available for Volvo (both intermediate tests between each production step and end of line tests).

#### EoL – Design review

Verification method	VM 54523319-58 v2

EoL (End of Line) test shall be agreed before B-release.

Pass criteria

Documented/stated compliance with requirements.

### In-Circuit Test

HWC requirement	REQ 54523319-37 v2

Each delivery unit shall pass an ICT (In-Circuit Test) or a combination of ICT and alternative verification methods. Full verification coverage is expected.

The verification process shall include power-up of the micro controller(s), connectors and all other components. The verification procedures used by the supplier shall be reviewed and agreed with the Volvo Group.

#### ICT – Design review

Verification method   VM 54523319-59 v2

Verification (ICT or combination) test shall be agreed before C-release.

Pass criteria

Documented/stated compliance with requirements.

To be confirmed in documentation and process description where a coverage matrix shall be presented and approved.

## Electrical Properties Low Voltage

TR information	INF 54523319-30 v2

- If the COTS component is connected to / powered only by the ADM, then it shall be tested as part of the ADM System, and these sections may not apply (to be discussed on a case-by-case basis).

- If the COTS components is powered directly from one of the vehicle platform power busses, then the COTS components shall be tested per the following tests, or else evidence shall be provided that it complies with the following tests (from the component manufacturer)

### Voltage rating

HWC requirement	REQ 54523319-38 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components powered by the ADM must be tested as part of the ADM system when the ADM is subject to this test.

The following table shows the COTS list components which are subject to Voltage Rating test.

Any COTS components directly powered by the vehicle platform busses must work within the following input voltage ranges:

| Parameter | Symbol | Condition | Unit | UN = 12 V |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | min | nom | max |  |
| Supply Voltage | Us_range | Maximum supply voltage range | V | –14 |  | +24 | No Damage within given time limits, FSC C |
|  | Us_normal | Normal supply voltage range, steady state | V | 0 | 14 | Us max | FSC C –> FSC A For the life of the component |
|  | Us_operating | Operating voltage range | V | Us min | 14 | Us max | Normal operation FSC A |
|  | UA | Test voltage | V | Us min | 14 | Us max | Generator operating. FSC A. Op mode 3.1 and 3.2 |
|  | UB | Test voltage | V | Us min | 12 | 12 | Generator not operating. FSC A. Op mode 2.1 and 2.2 |
|  |  | Reduced functionality | V | 6.0 | 6.5 | Us min | 6 V required for functions required during Engine Crank. 6,5 V is not a cost driver and can be used as nominal |
|  |  | Power down/up hysteresis | V |  | 0,5 |  |  |

| Parameter | Symbol | Condition | Unit | UN = 24 V |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | min | nom | max |  |
| Supply Voltage | Us_range | Maximum supply voltage range | V | –28 |  | +48 | No Damage within given time limits, FSC C |
|  | Us_normal | Normal supply voltage range, steady state | V | 0 | 28 | Us max | FSC A, B and C. For the life time of the component |
|  | Us_operating | Operating voltage range | V | Us min | 28 | Us max | Normal operation FSC A |
|  | UA | Test voltage | V | Us min | 28 | Us max | Generator operating. FSC A. Op mode 3.1 and 3.2 |
|  | UB | Test voltage | V | Us min | 24 | 24 | Generator not operating. FSC A. Op mode 2.1 and 2.2 |
|  |  | Reduced functionality | V | 8 | 8 | Us min | 8 V required for functions required during Engine Crank. 8 V is not a cost driver so can be used as nominal. |
|  |  | Power down/up hysteresis | V |  | 0,5 |  |  |

| Parameter | Symbol | Condition | Unit | UN = 3,3 V or 5,0 V |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | min | nom | max |  |
| Supply Voltage | Us_range | Maximum supply voltage range | V | 0 |  | Us max | No Damage, FSC C |
|  | Us_normal | Normal supply voltage range, steady state | V | 0 | 3.3 or 5.0 | Us max | FSC C –> FSC A For the life of the component |
|  | Us_operating | Operating voltage range | V | Us min | 3.3 or 5.0 | Us max | Normal operation FSC A |
|  | UA | Test voltage | V | Us min | 3.3 or 5.0 | Us max | Generator operating. FSC A. Op mode 3.1 and 3.2 |
|  | UB | Test voltage | V | Us min | 3.3 or 5.0 | Us max | Generator not operating. FSC A. Op mode 2.1 and 2.2 |

#### Slow decrease and increase of supply voltage – Test

TR information	INF 54523319-31 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

The purpose of the test is to catch circuits that aren't fully reset unless the voltage is less than 0,4–0,6 V.

Verification method	VM 54523319-60 v2

Test according to ISO 16750-2 Slow decrease and increase of supply voltage.

Pass criteria

FSC A is required inside the supply voltage range and FSC C is required outside the supply voltage range.

#### Momentary drop in supply voltage – Test

Verification method	VM 54523319-61 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

Test according to ISO 16750-2 Momentary drop in supply voltage.

Pass criteria

FSC A. If a reset is allowed, FSC C is OK.

#### Over-voltage – Test

Verification method	VM 54523319-62 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

Systems with 12 V nominal voltage

Test with supply voltage 18 V. Test duration: 1 h at Tmax – 20 °C in a hot air oven.

Test with supply voltage 24 V. Test duration: 5 min at room temperature.

Systems with 24 V nominal voltage

Test with supply voltage 36 V. Test duration: 1 h at Tmax – 20 °C in a hot air oven.

Test with supply voltage 48 V. Test duration: 2 minutes at room temperature.

Pass criteria

FSC C

#### Power supply interruption – Random test

TR information	INF 54523319-32 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

This test simulates a glitchy circuit in the voltage supply. In Case A, the glitchy circuit is on the supply side, while in Case B it is on the grounding side. This type of situation can occur when someone is connecting or disconnecting the battery, creating contact bounces.

Verification method	VM 54523319-63 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

Operational mode 3.2 with continuous monitoring

Test method

1. Place the component in a climate chamber.

2. Place the test equipment and test loads in a room-temperature environment (see GTINF18 General test conditions).

3. Connect the supply lines to the component according to Test Setup A with switch S2 closed.

4. Stabilize the component at Tmin. Apply Usnom (14 or 28 V) to the test voltage source Us.

5. Monitor the component performance and record the Ui voltage node at failure.

6. Touch the file surface U1 with the probe P1 connected to the component. Open switch S2. Move probe P1 along the file surface U1 back and forth with a speed of approx. 0–1 cm/s. The mechanical force of the probe P1 against the surface U1 shall be adjusted so that a relay chatter due to frequent connections/disconnections is heard. Continue for 5 minutes or until a malfunction is indicated by the monitoring equipment, then close S2 to maintain the condition with the supply voltage applied.

7. Investigate any found issues such as hardware failures, code runaway due to reset and/or watchdog malfunction. Record any triggered permanent fault codes.

Repeat steps 3 to 7 with Test Setup B

Equipment: File, any fine-cut file for metalworking. Relay, standard plug-in ISO Mini relay TE V23134-A0052-X278(12V), V23134-A0056-X4333(24V) or equivalent. Use a 12 V or 24 V suppressed (Rpar=10*Rcoil) coil relay,depending on the component Usnom.

Test setups for power supply interruption, Random

Pass criteria

FSC A

#### Reset behaviour at voltage drop – Test

Verification method	VM 54523319-64 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

Perform the test according to ISO 16750-2 Reset behaviour at voltage drop.

Operating mode 3.2

Then repeat the same test with the following remarks and modifications:

The component shall be connected to all real or equivalent loads for inputs and real loads for outputs.

The component shall be connected to all voltage-supply/ground pins and be soaked for 2h at Tmin unpowered before the test.

Remaining at Tmin, decrease the test voltage in steps of 5 % for each pulse from 100 % to 0 % of Usmin.

Verify the FSC A performance after each and every pulse of the 20 voltage drops after returning to Usmin.

Be very observant on that the reset is not appearing by itself more than one time after each drop.

If the internal voltage supply does not reach its steady-state value within the 5-second drop time, or if the 10-second function test time at Usmin is too short, the 5-second voltage-drop time or the 10-second function-test time shall be increased to fulfil this requirement of FSC A analysis after each voltage drop.

Pass criteria

FSC A

#### Reversed voltage – Test

Verification method	VM 54523319-65 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

Test according to ISO 16750-2 Reversed voltage

– Case 1 and Case 2 for 12 V systems.

– Case 2 for 24 V systems.

Correct loads shall be connected during the test. It should be stated whether the outputs shall be designed to handle the full current in reverse.

Pass criteria

FSC A

#### Superimposed alternating voltage – Test

Verification method	VM 54523319-66 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

Test according to ISO 16750-2 § 4.4.

The superimposed alternating voltage test shall be performed with severity 2.

Pass criteria

FSC A

#### Superimposed pulse voltage – Test

TR information	INF 54523319-33 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

This test simulates variations in Us within specification. The variations are caused by other loads in the vehicle. These can be due to the start of an electric motor that reduces the Us through high current consumption (inrush current) or a voltage increase caused by the deactivation of a motor and other inductive loads. A load controlled by a PWM circuit can also disturb the Us.

The sudden voltage drop can be detected by the ECU as a zero-current consumption by the load (the charge in the internal capacitors of the load might drive the load for a very short moment). The possible error can be that the ”Open circuit DTC” is sent out incorrectly. Similarly, a sudden increase in voltage might lead to a very short current rush from the ECU to the load (to charge the internal capacitors of the load). The possible error can be that the ”Short circuit DTC” is sent out incorrectly..

Verification method	VM 54523319-67 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

Test Definitions

Uo = Us + Up

Us = (Usmin + 2V) to (Usmax – 2 V) DC Voltage

Up = Square wave, – 1.0 to +1.0 V 50% duty cycle (2 V p–p)

Up-sweep frequency = 1 Hz to 4 kHz

Frequency sweep = Logarithmic

Frequency sweep duration = 120 s for 1 Hz to 4 kHz to 1 Hz

Operational mode = 3.2

Test Method

1. Place the component in a climate chamber and connect to the Uo output.

2. Raise the temperature to Tmax and stabilize.

3. Set Us = Usmax – 2,0 V.

4. Perform five continuous frequency sweep cycles while monitoring for permanent or intermittent faults.

5. Decrease Us by 1,0 V for 12 V components and 2,0 V for 24 V components.

6. Repeat steps (4) and (5) including Us = Usmin + 2,0 V.

7. Repeat steps (3) to (6) at Tmin.

Pass criteria

FSC A

### Power consumption

HWC requirement	REQ 54523319-39 v3

For any COTS components powered by of the vehicle power busses, Aurora shall provide documentation to Volvo, describing:

- The minimum power consumption, and under which conditions that shall be achieved,

- All different operational scenarios / conditions which drive their different power modes / power states,

- Appropriate vehicle platform requirements related to their power needs

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

#### Direct current supply voltage and current consumption – Test

Verification method	VM 54523319-68 v3

Requirement is applicable to vehicle-platform power interfaces. Other interfaces are n/a.

Voltage rating as defined in the ADM TR.

COTS components must be tested as part of the ADM system when the ADM is subject to this test.

Set the supply voltage as specified in GTREQ51 to all relevant inputs of the component.

Measure the current consumption of the component at room temperature and Usmin Usnom Usmax at Tmax as well as Usmin Usnom Usmax at Tmin, and Usmin Usnom Usmax according to the voltage rating requirement for all implemented power states. Also, measure the inrush current when switching from power OFF to ON and from the power states Sleep and Standby to Operational (if power states are supported).

Operating mode: (Operational) 3.2

Implemented Power state: see Power consumption requirement

Operating mode: Sleep to Operational 3.1/3.2

Operating mode: Standby to Operational 2.1/3.2

Pass criteria

–FSC A (If Sleep mode and/or Standby mode are applicable)

–FSC A and FSC C (If Sleep mode and/or Standby mode are not applicable)

### Grounding (If applicable)

Grounding tests are only applicable to COTS components which are directly powered by (or grounded to) the vehicle power buss(es).

(Not applicable if powered by / grounded via the ADM).

HWC requirement	REQ 54523319-40 v3

The following table shows the COTS list components which are subject to Grounding.

The component shall be able to withstand ground or supply voltage offsets.

If the casing of the Component is metallic, the power ground (pg) of the unit shall not be directly connected to the case with a low- DC-resistance connection.

Metallic sub-components shall not be electrically floating.

There shall be a capacitive coupling between the metallic casing and the Component power ground (C_case typically 100nF)

There shall be a high-ohm-value resistive path equal to 100 kOhm between the case and the power ground in the component.

The metallic casing itself shall have a direct connection to the vehicle chassis, cab or engine ground through metallic bolts or a wire connection.

#### Grounding – Test (If applicable)

TR information	INF 54523319-34 v3

This test shall verify the component’s ability to function properly when subjected to ground or supply voltage offsets.

The tests are applicable to all inputs on the component without signal return lines (signal_rtn) or outputs with at least one external connection to the tested component supply or ground line to the connected load. This means all I/Os on the component where voltage offsets occur in the supply or ground line(s). Voltage offsets will occur between different components within a vehicle.

Operational mode 3.2 with continuous monitoring is required.

Verification method	VM 54523319-69 v3

Test method for Ground offset

The ground offset shall be applied separately and simultaneously to each ground line of the I/O simulation device. The simulation device shall be constructed to meet the specified component loads. (Insert reference to components supplied or sufficient information for constructing equivalent loads for testing.)

1. Apply Usmin to the component.

2. Subject the applicable ground line of one representative I/O simulation device to a +1,5 V offset relative to the component ground.

3. Perform a function test operating mode 3.2 at Usmin and Usmax

4. Repeat step (2 and 3) for the next applicable I/O simulation device.

5. Repeat step (2 and 3) for all I/O simulation devices simultaneously.

6. Repeat steps (2) through (5) for a –1,5 V offset relative to the component ground.

7. Repeat steps (2) through (6) with Usmax instead of Usmin.

Test method for Supply offset

The supply voltage offset shall be applied separately and simultaneously to each supply line of the I/O simulation device. The simulation device shall be constructed according to the electrical interface documentation.

1. Apply Usmin to the component.

2. Subject the applicable supply line of one representative I/O simulation device to a –1,5 V offset relative to the component supply voltage (Uoffsx=+1,5 V).

3. Perform a function test operating mode 3.2 at Usmin and Usmax

4. Repeat step (2 and 3) for the next applicable I/O simulation device.

5. Repeat step (2 and 3) for all I/O simulation devices simultaneously.

6. Repeat steps (2) through (5) for a +1,5 V offset relative to the component supply voltage (Uoffsx = –1,5 V).

7. Repeat steps (2) through (6) with Usmax instead of Usmin.

Pass criteria

FSC A

### Open-circuit protection

HWC requirement	REQ 54523319-41 v2

All inputs and outputs (except battery supply and ground pins) shall be protected against open circuits.

#### Open-circuit protection – Test

Verification method	VM 54523319-70 v2

Test according to ISO 16750-2 Single line interruption and/or Multiple line interruption.

Pass criteria

The requirement is FSC A for interruption of signal circuits, except for interrupted circuits, where FSC C applies.

### Short-circuit protection

HWC requirement	REQ 54523319-42 v2

All inputs and outputs of the component shall be protected against short-circuits to battery and ground over the entire voltage and temperature range. This requirement includes hard and soft shorts to both supply and ground.

The component shall be designed to withstand the current required to trigger the power supply fuse or, as a minimum, a TBD A MINI blade fuse with a wire length of 1000 mm and a cable area of TBD mm2.

A hard short is defined as 0 Ohm for inputs and outputs. A soft short is defined as Ishort =1,35*Imax and Ishort=6*Imax for outputs, supply and ground lines.

The current through the component during a short-circuit shall be limited to avoid component damage. Shorting of inputs for power-to-ground and ground-to-battery will stress the harness instead of the component. Load returns and signal references that are grounded within the component shall also be protected against shorts to battery. Common active methods to protect Load returns and Signal references are serially connected PTC-resistors with the ground connection or a permanently switched-on, low-side, smart-FET driver to ground (with thermal short-circuit protection as a minimum). In case of no active protection of the Signal_rtn and the use of a copper track for the Signal reference to the Power_gnd connection, this track shall be designed to be the weakest point and shall be located on the top or bottom PCB layer so that the trace will act as a fuse in the event of a short to battery.

#### Short-circuit protection – Test

Verification method	VM 54523319-71 v2

Test according to ISO 16750-2 Short circuit protection, Signal circuits.

Test according to ISO 16750-2 Short circuit protection, Load circuits.

- Perform at min, nominal and max operating temperatures

- Use a representative wire length and cross-section (use 3 m as default length)

For electronic protection, apply for Loadx 110 % and 600 % of the maximum output-rated current.

For passive protection (fuse), apply for Loadx 135 % and 600 % of the maximum output-rated current with corresponding (maximum) time duration defined In ISO 8820.

Test cases and Operating modes

1 ⎯ Connected supply voltage and ground terminals

a. Outputs active => Operating mode 3.2

b. Outputs inactive => Operating mode 3.1 and 3.2

2 – Disconnected supply voltage terminals => Operating mode 1.2

3 – Disconnected ground terminals => Operating mode 1.2

Pass criteria

Test case 1

– Units with electronic protection. Requirement is FSC C for shorted circuits and FSC A for other circuits.

– Units with passive protection (fuse). Requirement is FSC D.

Test cases 2 and 3: FSC D

### HW start up time

HWC requirement	REQ 54523319-43 v2

The components internal electronics shall be fully functional after no longer than 50 ms after power up, ready to execute code.

#### HW start up time – Test

Verification method	VM 54523319-72 v2

The component is to be started up with intended SW or a similar test SW.

Pass criteria

The startup time shall be less than 50ms

## Electromagnetic Compatibility (EMC)

HWC requirement	REQ 54523319-44 v3

The tests shall be performed in accordance with Volvo STD 515-0003.

Severity index (SI): TBD based on joint discussions

- Loudspeaker:

- Multi-band antenna:

- GNSS antenna:

- Microphone:

Whenever Volvo STD 515-0003 is in conflict with another standard or test specification, Volvo

STD 515-0003 contains the applicable requirement.

Volvo STD 515-0003 shall be fulfilled within the load range specified for the DUT.

Test should be performed at expected worst case load and nominal load.

Any component powered by the ADM, it should be tested as part of the ADM system.

NOTE: Disturbances shall be applied at the relevant wires at the vehicle to ADM connection (interface) with the COTS components connected to the ADM.

NOTE: The measurements for conducted transients and emissions shall be done at the vehicle to ADM connection (interface) with the COTS components connected to the ADM.

The following table shows the COTS list components which are subject to EMC tests:

### Conducted Transient emission (If applicable)

HWC requirement	REQ 54523319-45 v3

From STD 515-0003 4.1

This procedure conforms to ISO 7637-2.

This test is intended to evaluate permanent interference, as well as switch-off and switch-on transients

sent from the DUT (Fig. 1). It is also intended to evaluate “braked/disconnected” electric motors (state 3).

Permanent interference shall be measured when switch S1 is closed and switch S2 is closed to state 2.

Transients are measured when opening or closing switch S1, and when switching S2 between its different states.

Exception: If a complete system is purchased from a supplier, the system might be regarded as a component,

and the conducted emission requirement shall apply to the system and not to the specific components.

#### Conducted transient emission Test case 1 (If applicable)

Verification method	VM 54523319-73 v3

The test set-up shall be in accordance with ISO 7637-2, section 4.3.

| S1 state | S2 state | Test goal | Times to be repeated |
| --- | --- | --- | --- |
| Permanently | Permanently | Waveforms sent | 1 |
| closed | closed to state 2 | by DUT |  |

Pass criteria:

| 24V system | 12V system |
| --- | --- |
| U2*= -75V < U2 < +50V | -50V < U2 < +50V |

(*U2 = Ua + Us, where Ua is the supply voltage and Us is the disturbance voltage or superimposed transient voltage)

U1 and U2 waveforms recordings shall be sent to Volvo for analysis.

#### Conducted transient emission Test case 2 (If applicable)

Verification method	VM 54523319-74 v3

The test set-up shall be in accordance with ISO 7637-2, section 4.3

| S1 state | S2 state | Test goal | Times to be repeated |
| --- | --- | --- | --- |
| Permanently | Switching from | Waveforms sent | 10 |
| closed | state 2 to state 1 | by DUT |  |

Pass criteria:

| 24V system | 12V system |
| --- | --- |
| U2*= -75V < U2 < +50V | -50V < U2 < +50V |

(*U2 = Ua + Us, where Ua is the supply voltage and Us is the disturbance voltage or superimposed transient voltage) U1 and U2 waveforms recordings shall be sent to Volvo for analysis.

#### Conducted transient emission Test case 3 (If applicable)

Verification method	VM 54523319-75 v3

The test set-up shall be in accordance with ISO 7637-2, section 4.3.

| S1 state | S2 state | Test goal | Times to be repeated |
| --- | --- | --- | --- |
| Permanently | Switching from | Waveforms sent | 10 |
| closed | state 1 to state 2 | by DUT |  |

Pass criteria:

| 24V system | 12V system |
| --- | --- |
| U2*= -75V < U2 < +50V | -50V < U2 < +50V |

(*U2 = Ua + Us, where Ua is the supply voltage and Us is the disturbance voltage or superimposed

transient voltage) U1 and U2 waveforms recordings shall be sent to Volvo for analysis.

#### Conducted transient emission Test case 4 (If applicable)

Verification method	VM 54523319-76 v3

The test set-up shall be in accordance with ISO 7637-2, section 4.3.

| S1 state | S2 state | Test goal | Times to be repeated |
| --- | --- | --- | --- |
| Opening (from | Permanently | Waveforms sent | 10 |
| ON to OFF) | closed to state 2 | by DUT, AN and Rs |  |

Pass criteria:

| 24V system | 12V system |
| --- | --- |
| U2*= -75V < U2 < +50V | -50V < U2 < +50V |

(*U2 = Ua + Us, where Ua is the supply voltage and Us is the disturbance voltage or superimposed

transient voltage) U1 and U2 waveforms recordings shall be sent to Volvo for analysis.

#### Conducted transient emission Test case 5 (If applicable)

Verification method	VM 54523319-77 v3

The test set-up shall be in accordance with ISO 7637-2, section 4.3

| S1 state | S2 state | Test goal | Times to be repeated |
| --- | --- | --- | --- |
| Closing (from | Permanently | Waveforms sent | 10 |
| OFF to ON) | closed to state 2 | by DUT, AN and Rs |  |

Pass criteria:

| 24V system | 12V system |
| --- | --- |
| U2*= -75V < U2 < +50V | -50V < U2 < +50V |

(*U2 = Ua + Us, where Ua is the supply voltage and Us is the disturbance voltage or superimposed

transient voltage) U1 and U2 waveforms recordings shall be sent to Volvo for analysis.

#### Conducted transient emission Test case 6 (If appliable)

Verification method	VM 54523319-78 v3

The test set-up shall be in accordance with ISO 7637-2, section 4.3.

| S1 state | S2 state | Test goal | Times to be repeated |
| --- | --- | --- | --- |
| Permanently | Switching from | Waveforms sent | 10 |
| closed | state 2 to state 3 | by DUT |  |

Pass criteria:

| 24V system | 12V system |
| --- | --- |
| U2*= -75V < U2 < +50V | -50V < U2 < +50V |

(*U2 = Ua + Us, where Ua is the supply voltage and Us is the disturbance voltage or superimposed

transient voltage) U1 and U2 waveforms recordings shall be sent to Volvo for analysis.

### Conducted RF emission – Voltage method (If applicable)

HWC requirement	REQ 54523319-46 v3

From STD 515-0003 4.2

Reference document

Components shall comply with the requirements defined in CISPR 25, Edition 05 or as agreed.

Field of application

This test is intended to evaluate the radio-frequency conducted disturbances generated by the DUT

and its power supply wiring.

Principal characteristics

- Frequency band [0,15–108 MHz]

- Peak detector and average detectors shall always be used.

- Peak and average detection are used for compliance with both average and peak limits.

Frequency bands outside of CISPR 25 bands are also included, not shaded in the table.

In case of overlap between bands, the lower limit shall apply.

#### Conducted RF emission (If applicable)

Verification method	VM 54523319-79 v3

Reference levels

The device(component)/system shall comply with the requirements specified in Table 4 (similar to CISPR Class 5)

Table 4

| Band |  | Frequency | Peak detector |  | Average detector |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | band (MHz) |  |  |  |  |  |
|  |  |  | Limit value dB | BW (kHz) | Limit value dB | BW (kHz) |  |
|  |  |  | (µV) |  | (µV) |  |  |
| LW | 1) | 0,15–0,30 | 70 | 9/10 | 50 | 9 |  |
|  | 2) | 0,30–0,53 | 70 | 9/10 | 50 | 9/10 |  |
| MW | 1) | 0,53–1,8 | 54 | 9/10 | 34 | 9/10 |  |
|  | 2) | 1,8–5,9 | 54 | 9/10 | 34 | 9/10 |  |
| SW | 1) | 5,9–6,2 | 53 | 9/10 | 33 | 9/10 |  |
|  | 2) | 6,2–26 | 53 | 9/10 | 33 | 9/10 |  |
| CB | 3) | 26–28 | 44 | 9/10 | 24 | 9/10 |  |
|  | 2) | 28–30 | 44 | 9/10 | 24 | 9/10 |  |
| VHF | 3) | 30–54 | 44 | 100/120 | 24 | 100/120 |  |
| TV band 11) |  | 41–88 | 34 | 100/120 | 24 | 100/120 |  |
| VHF | 3) | 68–87 | 38 | 100/120 | 18 | 100/120 |  |
| FM | 1) | 76–108 | 38 | 100/120 | 18 | 100/120 |  |

- Broadcast

- Free band

- Mobile services band

Pass criteria:

According to Table 4, no limit excess is allowed.

### Conducted transient susceptibility on power supply leads and I/O:s connected to power supply (If applicable)

HWC requirement	REQ 54523319-47 v3

From STD 515-0003 5.1

Conducted transient susceptibility on power supply leads and I/O:s connected to power supply.

This test is intended to check the immunity of the DUT to conducted electrical transients caused by

electronic and electric components. These interferences affect power supply lines and signal or sensor

input lines when these lines are directly or indirectly connected to the power supply lines.

Unless otherwise specified, the test shall be performed in accordance with ISO 7637-2 for pulses

1, 2, 3, and ISO 16750 for pulses 4, 5.

All pulses shall be superimposed on the lines of the device(component)/system connected to the

power supply.

All test pulses shall be characterized under no-load conditions, i.e. voltages are open-loop voltages.

| Table 5 FSC with respect to SI and pulse number Choose the severity index (SI) and read the functional status classification (FSC) attached to the pulse |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Pulse number | 1 | 2 | 3 | 4 | 5a 1) | 5c 2) |
| SI 1 | C | A | A | *) | A | A |
| SI 2 | C | B | B | *) | B | B |
| SI 3 | C | C | C | *) | C | C |

- This requirement is valid for 12 V systems only

- This requirement is valid for 24 V systems only

*)   This is decided on a case-by-case basis, depending on unit and application

Power outputs from the unit:

- Are not allowed to be over the specified Maximum supply voltage range

- Are not allowed to be below zero volt.

- Shall be within the specified Operating voltage range if either of the supplied unit or the unit that contains the power output shall fulfill FSC A or FSC B.

#### Test pulse 1 (If applicable)

Verification method	VM 54523319-80 v3

Scope: This pulse originates from the switching off of an inductive load in parallel with the tested component/system, e.g. electric valves without clamp diodes.

| Table 6 Parameters of test pulse 1 |  |  |
| --- | --- | --- |
| Parameter | 24-V system | 12-V system |
| Us (V) | -600 | -600 |
| Tr (µs) |  1 |  1 |
| T (ms) | 1 | 1 |
| T1 (s) | 5 | 5 |
| T2 (ms) | 200 | 200 |
| Ri () | 50 | 50 |
| Number of pulses | 1000 | 1000 |

Pass criteria:

FSC depending on severity index (see Table 5)

#### Test pulse 2 (If applicable)

Verification method	VM 54523319-81 v3

Scope: Pulse 2a originates from the switching off of an inductive load that is in series with the tested component/system, e.g. a DC motor. Pulse 2b simulates transients from a DC motor acting as a generator

after the ignition is switched off.

| Table 7 Parameters of test pulse 2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Test pulse 2a | Test pulse 2b |  |  |  |  |  |  |
|  |  | Nominal 12 V system |  |  | Nominal 24 V system |  |  |  |
|  |  | 10 |  |  | 20 |  |  |  |
| Us (V) | + 100 |  |  |  |  |  |  |  |
| Tr | 1 µs | 1 ms ± 0,5 ms |  |  |  |  |  |  |
| T | 0,05 ms | 2 s |  |  |  |  |  |  |
| T1 | 5 s | N/A |  |  |  |  |  |  |
| Ri | 10 | 0 Ω to 0,05 Ω |  |  |  |  |  |  |
| Number of | 1000 | 100 |  |  |  |  |  |
| pulses |  |  |  |  |  |  |  |
| T6 | N/A | 1 ms ± 0,5 ms |  |  |  |  |  |
| T12 | N/A | 1 ms ± 0,5 ms |  |  |  |  |  |

Pass criteria:

FSC depending on severity index (see Table 5)

#### Test pulse 3 (If applicable)

Verification method	VM 54523319-82 v3

Scope: This pulse simulates transients that occur during bouncing of a relay contact.

| Table 8 Parameters of test pulse 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Pulse 3a |  | Pulse 3a |  | Pulse 3b |  |  |
|  | 12-V system |  | 24-V system |  | 12-V and 24-V system |  |  |
| Us (V) | -150 |  | -200 |  | + 200 |  |  |
| Tr (ns) | ≤5 |  | ≤5 |  | ≤5 |  |  |
| Td (µs) | 0.1 |  | 0.1 |  | 0.1 |  |  |
| T1 (µs) | 100 |  | 100 |  | 100 |  |  |
| T4 (ms) | 10 |  | 10 |  | 10 |  |  |
| T5 (ms) | 100 |  | 100 |  | 100 |  |  |
| Ri  | 50 | 50 |  |  | 50 |  |
| Test duration | 1 h | 1 h |  |  | 1 h |  |

Pass criteria:

FSC depending on severity index (see Table 5)

#### Test pulse 4 (If applicable)

Verification method	VM 54523319-83 v3

Scope: This pulse originates from voltage drop on main supply due to cranking.

| Table 9 Parameters of test pulse 4 |  |  |
| --- | --- | --- |
| Parameter | 12V system | 24V system |
| U1* (V) | 12 | 24 |
| U2 (V) | 5 | 8 |
| U3 (V) | 10 | 18 |
| U4 (V) | 6 | 12 |
| Tr (ms) | 5 | 10 |
| Tf (ms) | 10 | 10 |
| T1 (ms) | 40 | 100 |
| T2 (ms) | 2 | 2 |
| T3 (ms) | 100 | 100 |
| T4 (s) | 20 | 20 |

*U1: Value corresponding to battery state of charge of 50 %.

Pass criteria:

FSC shall be decided case by case depending on unit and application.

#### Test pulse 5 (If applicable)

Verification method	VM 54523319-84 v3

Scope: This pulse may be generated using a charging alternator connected to a battery with an appropriate load in parallel. The alternator speed and load should then be varied and the corresponding load dump pulse logged using an oscilloscope, the component/system not connected, to find the appropriate combination for producing the desired test pulse. The component can then be connected and the test performed.

A total of five pulses shall be applied for each pulse type (a and c) tested. A period of 60 seconds shall pass between the pulses. Ua is the supply voltage according to Section 3.1.

| Table 10 Parameters of test pulse 5 |  |  |  |
| --- | --- | --- | --- |
| Test Pulse |  | Pulse 5a 1) | Pulse 5c 1) |
| Applicability |  | 12-V system | 24-V system |
| Parameters | Us (V) | 25.8 | 29.5 |
|  | Tr (ms) | ≤ 5 | ≤ 5 |
|  | T1 (ms) | 160 | 160 |
|  | T (ms) | 600 | 600 |
|  | Ri (Ω) | 0.7 | 1.8 |

1) Alternator with internal zener diode.

Pass criteria:

FSC depending on severity index (see Table 5)

### Conducted transient susceptibility on signal leads

HWC requirement	REQ 54523319-48 v2

From STD 515-0003 5.2

Scope: This test is intended to ensure the immunity to transient transmission by inductive and capacitive coupling via vehicle signals or sensor cables.

A capacitive coupling clamp (CCC) is used to test capacitive coupling with fast pulses.

An inductive coupling clamp (ICC) is used to test inductive coupling with slow pulses.

The device/system shall withstand exposure to transients coupled to all signal lines. Test pulses and test methods are defined in ISO 7637-3.

#### Conducted transient susceptibility on signal leads fast

Verification method	VM 54523319-85 v2

Fast-transient test pulses 3a and 3b

Parameters are identical to section 5.1.3 of this document (pulse 3 applied on supply leads) with the

exception of pulse amplitude and number of pulses as follows:

Pulse 3a: Us = -200 V, test time 10 minutes

Pulse 3b: Us = +200 V, test time 10 minutes

Pass criteria:

Irrespective of SI classification, FSC A is required for both pulses.

#### Conducted transient susceptibility on signal leads Slow

Verification method	VM 54523319-86 v2

Slow-transient test pulses positive and negative 2a

According to ISO 7637-3

Test level IV

Pulse positive 2a: Us = +10 V, test time 5 minutes

Pulse negative 2a: Us = -10 V, test time 5 minutes

Pass criteria:

Irrespective of SI classification, FSC A is required for both pulses.

### Power supply quality (If applicable)

HWC requirement	REQ 54523319-49 v3

From 515-0003 5.3

Test method

A programmable power supply or a high-power operational amplifier and a signal generator may be used to generate the “saw-tooth” supply voltage. It must be able to supply both the static and dynamic currents required by the tested component/system.

As an alternative, an alternator and a partly charged battery may be used to generate the test voltage

specified in section 5.3.2. A high-power solid-state relay may be used to generate the cuts required in

section 5.3.3.

An ordinary power supply can be used to generate the test voltage specified in section 5.3.4.

Power outputs from the unit:

- Are not allowed to be over the specified Maximum supply voltage range

- Are not allowed to be below zero volt.

- Shall be within the specified Operating voltage range if either of the supplied unit or the unit that contains the power output shall fulfill FSC A or FSC B.

#### Power supply with battery (If applicable)

Verification method	VM 54523319-87 v3

This test is intended to check the immunity of the device(component)/system to minimum and maximum voltages of the power system due to the regulation of the alternator.

The voltage is that encountered in a commercial vehicle, with the following characteristics:

| Table 11 Parameters of test voltage, power-supply quality |  |  |
| --- | --- | --- |
|  | With Battery |  |
| Parameter | 12 V | 24 V |
| U1 [V] | 16 | 31 |
| U2 [V] | 14.2 | 28.5 |
| U3[V] | 11 | 26 |
| Tr [µs] | 20 | 20 |
| Td [µs] | 160 | 160 |
| Test duration | 1 h | 1 h |

Pass criteria:

FSC A

#### Power supply without battery (If applicable)

Verification method	VM 54523319-88 v3

This test is intended to check the immunity of the device(component)/system to minimum and maximum voltages of the power system due to the regulation of the alternator.

The voltage is that encountered in a commercial vehicle, with the following characteristics:

| Table 11 Parameters of test voltage, power-supply quality |  |  |
| --- | --- | --- |
|  | Without Battery |  |
| Parameter | 12 V | 24 V |
| U1 [V] | 18 | 30 |
| U2 [V] | 13 | 25 |
| U3[V] | 8 | 20 |
| Tr [µs] | 20 | 20 |
| Td [µs] | 160 | 160 |
| Test duration | 1 h | 1 h |

Pass criteria:

FSC A

#### Immunity to micro power cuts (If applicable)

Verification method	VM 54523319-89 v3

Scope: This test comes from problems with logic control circuits due to short-duration power cuts causing software-related problems.

The power supply of the tested component/system shall undergo micro power cuts as stipulated in the component TR. Unless otherwise specified, tests with a power cut duration (tinterrupt) of 100, 200 and 400 μs shall be applied 5 times, with a >1 s delay between each power cut. Waveform transition times shall be approximately 10 μs and the voltage drop shall be from nominal operating voltage, Usnom to 0 V. For illustration of the test pulse, see the figure below.

The test harness connecting the DUT to the test fixture and transient pulse generator shall be

≤ 2000 mm in length.

During the micro power cut, the supply line shall be in open circuit.

Pass criteria:

A component/system shall be capable of uninterrupted operation when micro power cuts are applied (FSC A).

#### Immunity to high-voltage supply (If applicable)

Verification method	VM 54523319-90 v3

Scope: This test comes from battery chargers/starters used in combination with batteries in poor condition.

This will result in a rise of the supply voltage.

A component shall be immune to high voltage in the supply lines for a certain amount of time. This simulates a quick charger/starter.

Table 12 Requirements per electrical system and test

| Table 12 Requirements per electrical system and test |  |  |  |
| --- | --- | --- | --- |
| 12-V systems |  |  |  |
| Test name | U, supply | T, duration | Requirement |
| Charger/starter | 24 V | 5 min | FSC D |
| 24-V systems |  |  |  |
| Test name | U, supply | T, duration | Requirement |
| Charger/starter | 48 V | 2 min | FSC D |

Pass criteria:

FSC D

### Radiated emission CV (Complete Vehicle)

HWC requirement	REQ 54523319-50 v2

From 515-0003-6.1

#### Radiated emissions measured at the foot of the antenna – Test (If applicable)

Verification method	VM 54523319-91 v3

Reference document

The components shall comply with the requirements defined in CISPR 25, Edition 05 or as agreed.

Field of application

This test is intended to evaluate the level of electromagnetic disturbance received by the vehicle’s antennas.

Principal characteristics:

- Frequency band [150 kHz – 3,0 GHz]

- Peak and average detectors shall always be used.

- Peak and average detection are used to measure compliance with both average and peak limits.

Comments:

Frequency bands outside of CISPR 25 bands are also included, not shaded in the table.

In the 30–3000 MHz range with 100/120 kHz bandwidth, the measurements using average detector can be conducted with a bandwidth of 9/10 kHz if the ambient electromagnetic noise level measured at 100/120 kHz is not at least 6 dB lower than the applicable limit.

Unless otherwise defined in CISPR 25, Edition 05 or as agreed, the measurement time (receiver) or sweep time (spectrum analyzer) shall be increased if low recursive frequency must be measured.

Based on the type of possible receivers (depending on vehicle range and region), the tests can be carried out with the specific frequency bands specified by the EMC responsible at Volvo. This will be decided on a case-by-case basis.

In case of an overlap between bands, the lower limit shall apply.

Radiated levels

To protect on-board receivers, the disturbance voltage at the end of the antenna cable shall not exceed the values in table 13.

Table 13

| Band |  |  | Frequency band (MHz) | Peak detector |  | Average detector |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Limit value dB(µV) | BW (kHz) | Limit value dB(µV) | BW (kHz) |
| LW |  | 1) | 0,15–0,30 | 26 | 9/10 | 20 | 9/10 |
| 2) |  |  | 0,30–0,53 | 26 | 9/10 | 20 | 9/10 |
| MW |  | 1) | 0,53–1,8 | 22 | 9/10 | 10 | 9/10 |
| 2) |  |  | 1,8–5,9 | 22 | 9/10 | 10 | 9/10 |
| SW |  | 1) | 5,9–6,2 | 22 | 9/10 | 10 | 9/10 |
| 2) |  |  | 6,2–26 | 22 | 9/10 | 10 | 9/10 |
| CB |  | 3) | 26–28 | 22 | 9/10 | 0 | 9/10 |
| 2) |  |  | 28–30 | 22 | 9/10 | 0 | 9/10 |
| VHF |  | 3) | 30–54 | 20 | 100/120 | 0 | 100/120 |
| TV Band 1 |  | 1) | 41–88 | 16 | 100/120 | 6 | 100/120 |
| VHF |  | 3) | 68–87 | 20 | 100/120 | 0 | 100/120 |
| FM |  | 1) | 76–108 | 26 | 100/120 | 6 | 100/120 |
| 2) |  |  | 108–137 | 26 | 100/120 | 6 | 100/120 |
| VHF |  | 3) | 137–175 | 20 | 100/120 | 0 | 100/120 |
| TV Band 3 |  | 1) | 174–230 | 16 | 100/120 | 6 | 100/120 |
| DAB 3 |  | 1) | 171–245 | 10 | 100/120 | 0 | 100/120 |
| 2) |  |  | 245–300 | 20 | 100/120 | 6 | 100/120 |
| RKE |  | 3) | 300–330 | 20 | 100/120 | 6 | 100/120 |
| 2) |  |  | 330–380 | 20 | 100/120 | 6 | 100/120 |
| RKE |  | 3) | 420–450 | 20 | 100/120 | 6 | 100/120 |
| UHF |  | 3) | 380–512 | 20 | 100/120 | 0 | 100/120 |
| DTTV |  | 1) | 470–770 | 20 | 100/120 | 10 | 100/120 |
| TV Band 4/5 |  | 1) | 468–944 | 16 | 100/120 | 6 | 100/120 |
| UHF |  | 3) | 820–960 | 20 | 100/120 | 0 | 100/120 |
| GSM 800 | 3) |  | 860–895 | 26 | 100/120 | 6 | 100/120 |
| GSM 900 | 3) |  | 925–960 | 26 | 100/120 | 6 | 100/120 |
| 2) |  |  | 960–1447 | 26 | 100/120 | 6 | 100/120 |
| DAB L BAND 1) |  |  | 1447–1494 | 10 | 100/120 | 0 | 100/120 |
| 2) |  |  | 1494–1567 | 20 | 100/120 | 0 | 100/120 |
| GPS | 3) |  | 1567–1583 | 20 | 100/120 | 0 | 100/120 |
| 2) |  |  | 1583–1803 | 26 | 100/120 | 6 | 100/120 |
| GLONASS | 3) |  | 1591–1616 | 20 | 100/120 | 0 | 100/120 |
| GSM 1800 | 3) |  | 1803–1882 | 26 | 100/120 | 6 | 100/120 |
| GSM 1900 | 3) |  | 1850–1990 | 26 | 100/120 | 6 | 100/120 |
| UMTS (3G) | 3) |  | 1900–2172 | 26 | 100/120 | 6 | 100/120 |
| 2) |  |  | 2172–2320 | 26 | 100/120 | 6 | 100/120 |
| SDARS | 1) |  | 2320–2345 | 16 | 100/120 | 6 | 100/120 |
| 2) |  |  | 2345–2400 | 16 | 100/120 | 6 | 100/120 |
| Bluetooth /WLAN 3) |  |  | 2400–2500 | 26 | 100/120 | 6 | 100/120 |
| 2) |  |  | 2500–3000 | 26 | 100/120 | 6 | 100/120 |

1)   Broadcast

2)   Free band

3)   Mobile services band

Pass criteria:

According to Table 13, no limit excess is allowed.

#### Radiated emissions measured in accordance with ECE R10 - Test (If applicable)

Verification method	VM 54523319-92 v3

The test methods are in accordance with ECE R10 with the following setup:

- Frequency band [30 MHz – 2 GHz]

- Horizontal and vertical polarization

- Limits according to table 14 and table 15

References levels

The vehicle shall comply with the requirements stated in table 14 and table 15.

Measurements at 3 meters or 10 meters:

| Table 14 Peak and average when antenna-vehicle separation is 3 meters |  |  |  |
| --- | --- | --- | --- |
| Frequency (MHz) | Peak detector limit (dBµV/m) | Average detector limit (dBµV/m) |  |
| 30–75 | 54 | 32 |  |
| 75–400 | 54 + 15,13·log (F/75) | 32 + 15,13·log (F/75) |  |
| 400–2000 | 65 | 43 |  |

| Table 15 Peak and average when antenna-vehicle separation is 10 meters |  |  |
| --- | --- | --- |
| Frequency (MHz) | Peak detector limit (dBµV/m) | Average detector limit (dBµV/m) |
| 30–75 | 44 | 22 |
| 75–400 | 44 + 15,13·log (F/75) | 22 + 15,13·log (F/75) |
| 400–2000 | 55 | 33 |

Pass criteria:

According to Table 14 and 15, no limit excess is allowed.

### Radiated emission (If applicable)

HWC requirement	REQ 54523319-51 v3

From 515-0003 6.2

Reference document

The components shall comply with the requirements defined in CISPR 25, Edition 05 or as agreed.

Field of application

This test is intended to evaluate radio frequency emitted disturbances generated by the DUT and its wiring.

#### Radiated emission ALSE (If applicable)

Verification method	VM 54523319-93 v3

Principal characteristics:

- Peak detector and average detector shall always be used. Peak and average detection are used to measure compliance with both average and peak limits.

- Frequency band [150 kHz – 3,0 GHz].

- Frequency bands outside of CISPR 25 bands are included, not shaded in the table.

- In the range 30–3000 MHz with 100/120 kHz bandwidth, the measurements using average detector can be conducted with a bandwidth of 9/10 kHz if the ambient electromagnetic noise level measured at 100/120 kHz is not at least 6 dB lower than the applicable limit.

- Unless otherwise defined in CISPR 25, Edition 05 or as agreed, the measurement time (receiver) or sweep time (spectrum analyzer) shall be increased if low recursive frequency shall be measured.

- In case of an overlap between bands, the lower limit shall apply.

In a component test, the actuator, sensors, switches, etc., connected to the tested component/system shall, as far as possible, be the same as those used in production.

In a component test, the component/system with its actuator, sensors, switches, etc., shall be connected to the ground plane in the same way as the vehicle installation.

References levels

The device(component)/system shall comply with the requirements in table 16.

| Table 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Frequency |  | Peak detector |  |  |  |  | Average detector |  |  |  |  |
|  |  | band (MHz) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Limit value |  |  | BW (kHz) |  | Limit value |  |  | BW (kHz) |  |
|  |  |  |  | dB(µV/m) |  |  |  |  | dB(µV/m) |  |  |  |  |
| LW1 | 1) | 0,15–0,30 |  | 46 |  |  | 9/10 | 26 |  |  |  | 9/10 |  |
|  | 2) | 0,30–0,53 | 46 |  |  |  | 9/10 | 26 |  |  | 9/10 |  |  |
| MW | 1) | 0,53–1,8 |  | 40 |  |  | 9/10 | 20 |  |  |  | 9/10 |  |
|  | 2) | 1,8–5,9 | 40 |  |  |  | 9/10 | 20 |  |  | 9/10 |  |  |
| SW | 1) | 5,9–6,2 |  | 40 |  |  | 9/10 | 20 |  |  |  | 9/10 |  |
|  | 2) | 6,2–26 | 40 |  |  |  | 9/10 | 20 |  |  | 9/10 |  |  |
| CB | 3) | 26–28 |  | 40 |  |  | 9/10 | 20 |  |  |  | 9/10 |  |
|  | 2) | 28–30 | 40 |  |  |  | 9/10 | 20 |  |  | 9/10 |  |  |
| VHF | 3) | 30–54 |  | 40 |  |  | 100/120 | 20 |  |  |  | 100/120 |  |
| TV Band 1 | 1) | 41–88 |  | 28 |  |  | 100/120 | 18 |  |  |  | 100/120 |  |
| VHF | 3) | 68–87 |  | 35 |  |  | 100/120 | 15 |  |  |  | 100/120 |  |
| FM | 1) | 76–108 |  | 38 |  |  | 100/120 | 18 |  |  |  | 100/120 |  |
|  | 2) | 108–137 | 38 |  |  |  | 100/120 | 18 |  |  | 100/120 |  |  |

| Band |  |  | Frequency |  |  |  | Peak detector |  |  |  |  |  | Average detector |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | band (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Limit value |  |  |  | BW (kHz) |  | Limit value |  |  | BW (kHz) |  |  |
|  |  |  |  |  |  |  | dB(µV/m) |  |  |  |  |  | dB(µV/m) |  |  |  |  |  |
| VHF | 3) |  | 137–175 |  |  |  | 35 |  |  |  | 100/120 |  | 15 |  |  |  |  | 100/120 |
| TV Band 3 | 1) |  | 174–230 |  |  |  | 32 |  |  |  | 100/120 |  | 22 |  |  |  |  | 100/120 |
| DAB 3 | 1) |  | 171–245 |  |  |  | 26 |  |  |  | 100/120 |  | 16 |  |  |  |  | 100/120 |
| RKE | 3) |  | 300–330 |  |  |  | 32 |  |  |  | 100/120 |  | 18 |  |  |  |  | 100/120 |
|  |  | 2) | 330–380 | 32 |  |  |  |  |  |  |  | 100/120 |  | 18 |  |  | 100/120 |  |  |
| RKE |  | 3) | 420–450 |  |  | 32 |  |  |  |  |  | 100/120 |  | 18 |  |  | 100/120 |  |  |
| UHF |  | 3) | 380–512 |  |  | 38 |  |  |  |  |  | 100/120 |  | 18 |  |  | 100/120 |  |  |
| DTTV |  | 1) | 470–770 |  |  | 45 |  |  |  |  |  | 100/120 |  | 35 |  |  | 100/120 |  |  |
| TV Band 4/5 |  | 1) | 468–944 |  |  | 41 |  |  |  |  |  | 100/120 |  | 31 |  |  | 100/120 |  |  |
| UHF |  | 3) | 820–960 |  |  | 44 |  |  |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
| GSM 800 |  | 3) | 860–895 |  |  | 44 |  |  |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
| GSM 900 |  | 3) | 925–960 |  |  | 44 |  |  |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
|  |  | 2) | 960–1447 | 44 |  |  |  |  |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
| DAB L BAND |  | 1) | 1447–1494 |  | 28 |  |  |  |  |  |  | 100/120 |  | 18 |  |  | 100/120 |  |  |
|  |  | 2) | 1494–1567 | 44 |  |  |  |  |  |  |  | 100/120 |  | 18 |  |  | 100/120 |  |  |
| GPS |  | 3) | 1567–1583 |  |  |  |  | 44 |  |  |  | 100/120 |  | 10 |  |  | 100/120 |  |  |
|  |  | 2) | 1583–1803 | 44 |  |  |  |  |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
| GLONASS |  | 3) | 1591–1616 |  |  |  |  | 44 |  |  |  | 100/120 |  | 10 |  |  | 100/120 |  |  |
| GSM 1800 |  | 3) | 1803–1882 |  |  |  |  | 44 |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
| GSM 1900 |  | 3) | 1850–1990 |  |  |  |  | 44 |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
| UMTS (3G) |  | 3) | 1900–2172 |  |  |  |  | 44 |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
|  |  | 2) | 2172–2320 | 44 |  |  |  |  |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
| SDARS |  | 1) | 2320–2345 |  |  |  |  | 34 |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
|  |  | 2) | 2345–2400 | 44 |  |  |  |  |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
| Bluetooth /WLAN 3) |  |  | 2400–2500 |  |  |  |  | 44 |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |
|  |  | 2) | 2500–3000 | 44 |  |  |  |  |  |  |  | 100/120 |  | 24 |  |  | 100/120 |  |  |

- Broadcast

- Free band

- Mobile services band

Pass criteria:

According to Table 16, no limit excess is allowed.

### General - Radiated susceptibility (If applicable)

HWC requirement	REQ 54523319-52 v3

From 515-0003 7.1

The frequency range where the radiated susceptibility requirements apply is 150 kHz – 3 GHz, with the additional radar frequency bands up to 12,4 GHz.

- Substitution method with the conservation of peak level

- Semi-anechoic or anechoic chamber shall be used

Modulation vs frequency range:

- CW in the 150 kHz – 3 GHz range

- AM in the 150 kHz – 800 MHz range

- PM1 in the 800 MHz – 3 GHz range

- PM2 in the 1,15–1,45 GHz range, L-band

- PM2 in the 2,6–3,2 GHz range, S-band

- PM2 in the 5,2–5,9 GHz range, C-band

- PM2 in the 8,2–12,4 GHz range, X-band

According to ISO 11451-1 and ISO 11452-1

According to ISO 11451-1 and ISO 11452-1

- Non-modulated signal: continuous wave (CW)

- Amplitude modulated (AM): 1 kHz 80 %

- Pulse-modulated 1 (PM1): Ton 577 µs, period 4600 µs

- Pulse-modulated 2 (PM2): Ton 3 µs, period 3333 µs

To confirm that the vehicle/component meets the requirements in this section, the vehicle/component shall be tested with the following maximum frequency steps:

- From 150 kHz to 1 MHz: step 30/decade

- From 1 MHz to 20 MHz: step 0,5 MHz

- From 20 MHz to 220 MHz: step 1 MHz

- From 220 MHz to 500 MHz: step 2 MHz

- From 500 MHz to 1 GHz: step 5 MHz

- From 1 GHz to 3 GHz: step 20 MHz

- L-band, from 1,15–1,45 GHz: step 5 MHz

- S-band, from 2,6–3,2 GHz: step 5 MHz

- C-band, from 5,2–5,9 GHz: step 10 MHz

- X-band, from 8,2–12,4 GHz: step 20 MHz

At each frequency, the field amplitude shall be applied as follows:

- From 0 V/m to the maximum

- Constant during X seconds

- From maximum to 0 V/m.

The amplitude modulation is effective during the constant part of the cycle. Time X shall be sufficient to

allow the device(component)/system under test to react to the interference and for any reaction to be recorded; X shall never be less than 2 seconds.

Vertical and horizontal polarization shall be used above 20 MHz.

Table 17.  FSC requirements and test levels

| Severity index |  | SI 1 | SI 2 | SI 3 |
| --- | --- | --- | --- | --- |
| Test level | III | A | B | D |
|  | II | A | A | C |
|  | I | A | A | A |

Observe the defined severity index (SI) as defined in the component’s technical regulation.

Observe the defined functional status classification (FSC) for each level.

Carry out the different test levels and check FSC compliance.

#### Radiated susceptibility CV (Complete Vehicle) - Test

Verification method	VM 54523319-94 v2

A complete vehicle test shall conform to ISO 11451-2 and ISO 11451-3.

Table 18 Test levels with respect to test methods

| Test method | Applicable standard | Test level (V/m) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Level I | Level II | Level III |
| Vehicle test, far field | ISO 11451-2 | 30 | 50 | 100 |
| On-board transmitter simulation | ISO 11451-3 | See tables A.1 and A.2 of ISO 11451-3 |  |  |

#### Radiated susceptibility component - Test (If applicable)

Verification method	VM 54523319-95 v3

The components shall comply with the following requirements.

The applicable frequency range for this test method is 150 kHz – 3000 MHz, and L-, S-, C-, X-bands

(including both far field and BCI test methods). Testing over the full frequency range could require

different field-generating devices, but this does not imply that testing of overlapping frequency ranges

is required. Only test methods described in table 19 are accepted.

| Table 19 Test levels with respect to test methods |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test method | Applicable | Test level |  |  |  |
|  | standard | Level I | Level II | Level III |  |
| Far field (V/m), 200–3000 MHz | ISO 11452-2 | 30 | 50 | 100 |  |

Pass criteria:

FSC depending on severity index (see Table 17)

#### Radiated susceptibility component test -Far field Radar (If applicable)

Verification method	VM 54523319-96 v3

The components shall comply with the following requirements.

The applicable frequency range for this test method is 150 kHz – 3000 MHz, and L-, S-, C-, X-bands (including both far field and BCI test methods). Testing over the full frequency range could require different field-

generating devices, but this does not imply that testing of overlapping frequency ranges is required.

Only test methods described in table 20 are accepted.

Table 20  Test levels with respect to test methods

| Test method | Applicable | Test level |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | standard | Level I | Level II | Level III |  |
| Far field (V/m), L-band | ISO 11452-2 | 200 | 600 | N/A |  |
| Far field (V/m), S-band | ISO 11452-2 | 100 | 200 | N/A |  |
| Far field (V/m), C-band | ISO 11452-2 | 100 | 200 | N/A |  |
| Far field (V/m), X-band | ISO 11452-2 | 100 | 200 | N/A |  |

Pass criteria:

FSC depending on severity index (see Table 17)

#### Radiated susceptibility component test -BCI (If applicable)

Verification method	VM 54523319-97 v3

The components shall comply with the following requirements.

The applicable frequency range for this test method is 150 kHz – 3000 MHz, and L-, S-, C-, X-bands (including both far field and BCI test methods). Testing over the full frequency range could require different field-generating devices, but this does not imply that testing of overlapping frequency ranges is required. Only test methods described in table 21 are accepted.

Table 21  Test levels with respect to test methods

| Test method | Applicable | Test level |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | standard | Level I | Level II | Level III |  |
| BCI test* (mA), 150 kHz – 400 MHz | ISO 11452-4 | 60 | 100 | 200 |  |

*)When using the closed-loop method with limitation of forward power, K = 4 shall be used. When using substitution method, all 3 positions (150 mm, 450 mm, 750 mm) shall be used. If multiple harnesses or connectors are used, a test shall be performed for each harness/connector.

Pass criteria:

FSC depending on severity index (see Table 17)

#### Simulated portable transmitter (If applicable)

Verification method	VM 54523319-98 v3

This test is applicable for electrical and electronic components that can be located close to handheld transmitters (e.g. cellular phone). For each component, Volvo shall evaluate and determine if this test needs to be performed. This test should be performed according to ISO 11452-9, excluding the frequency ranges for “10 m”, “2 m” and “IEEE 802.11a”.

Pass criteria:

According to Table A.1 of ISO 11452-9

### Immunity to electrostatic discharge

HWC requirement	REQ 54523319-53 v2

From 515-0003 8

This test is intended to check the immunity of components to electrostatic discharges produced:

- by operators during storage, handling, assembly and maintenance. Unpowered test is required.

- by occupants in or near the vehicle and customer service operation. Powered test is required.

The electrostatic discharge immunity test procedure shall be conducted using ISO 10605 with the following

conditions and requirements.

#### Immunity to ESD Handling

Verification method	VM 54523319-99 v2

This test is intended to check the immunity of components to electrostatic discharges produced:

- by operators during storage, handling, assembly and maintenance. Unpowered test is required.

- by occupants in or near the vehicle and customer service operation. Powered test is required.

The electrostatic discharge immunity test procedure shall be conducted using ISO 10605 with the following

conditions and requirements.

| Tests | Direct contact Discharge sequence | Air contact Discharge sequence | Specifications |
| --- | --- | --- | --- |
| Component test: Unpowered tests (handling) | ± 4 / 8 KV On each connector pin2) and conductive surface of the device (component) / system | ± 4 / 8 / 15 / (30)1) KV On all accessible points and other surfaces of the device (component) / system | Supplier test Sequence with a series of 10 discharges for each negative and positive level (from the lowest level to the highest level) Intervals between each discharge: 5s minimum Network: R = 330 Ω, C = 150 pF |
|  | Requirements: FCS A AFTER RECONNECTION |  |  |

- Only for pyrotechnic components.

- For testing pins in contact mode, an extension line for easy discharge can be used. The line shall be no longer than 25 mm.

Pass criteria:

FSC A after reconnection

#### Immunity to ESD Powered

Verification method	VM 54523319-100 v2

This test is intended to check the immunity of components to electrostatic discharges produced:

- by operators during storage, handling, assembly and maintenance. Unpowered test is required.

- by occupants in or near the vehicle and customer service operation. Powered test is required.

The electrostatic discharge immunity test procedure shall be conducted using ISO 10605 with the following

conditions and requirements.

| Tests | Direct contact Discharge sequence | Air contact Discharge sequence | Specifications |
| --- | --- | --- | --- |
| Component test: Powered-up tests (live and loaded conditions) | ± 4 / 8 KV (Device(component)/system and remote inputs/outputs)2) On all points accessible by vehicle occupants during maintenance and operation (determined in the test plan) | ± 4 / 8 / 15 / (25)1) KV (Device(component)/ system and remote inputs/outputs)2) On all points accessible by vehicle occupants during maintenance and operation (determined in the test plan) | Supplier test Sequence with a series of 10 discharges for each negative and positive level (from the lowest to the highest level) Intervals between each discharge: 5 s minimum Network: R = 330 Ω, C = 330 pF (150 pF)a |
|  | Requirements: ± 4 / 8 KV: FSC A for severity indices 1 and 2, FSC B for severity index 3 ± 15 / (25)1) KV: FSC B for severity indices 1 and 2, FSC C for severity index 3 |  |  |

- Only for pyrotechnic components.

- For powered-up tests, the device(component)/system shall be connected to all inputs/outputs necessary for functional testing, as well as controls, actuators, sensors, etc.

All discharge sequences also apply to:

- All devices(components)/systems directly connected to the unit and accessible by vehicle occupants during maintenance and operation, as well as remote control, switches, displays, harness, connectors, etc.

- All inputs/outputs that are connected to communication buses and regarded as accessible points (e.g.: diagnostic plug, OBD plug). In that case C = 150 pF; note a

- All devices(components)/systems directly connected to the unit may be subject to ESD indirectly by a charged source (e.g., wheel speed sensor, external temperature sensor, etc.)

In agreement with Volvo, if a periphery is not available for the test, the connecting lines shall be tested directly with discharges.

Pass criteria:

± 4 / 8 KV: FSC A for severity indices 1 and 2, FSC B for severity index 3

± 15 / (25)1) KV: FSC B for severity indices 1 and 2, FSC C for severity index 3

#### Immunity to ESD Complete vehicle

Verification method	VM 54523319-101 v2

This test is intended to check the immunity of components to electrostatic discharges produced:

- by operators during storage, handling, assembly and maintenance. Unpowered test is required.

- by occupants in or near the vehicle and customer service operation. Powered test is required.

The electrostatic discharge immunity test procedure shall be conducted using ISO 10605 with the following conditions and requirements.

| Vehicle test | If needed Test plan, specifications and requirements defined by the EMC engineering department Contact discharge: ± 4 / 8 KV Air discharge: ± 4 / 8 / 15 / (25)1) KV | Vehicle manufacturer Test Network: R = 330 Ω, C = 150 / 330 pF2) |
| --- | --- | --- |
|  | Requirements: ± 4 / 8 KV: FSC A for severity indices 1 and 2, FSC B for severity index 3 ± 15 / (25)1) KV: FSC B for severity indices 1 and 2, FSC C for severity index 3 |  |

- Only for pyrotechnic components.

- C = 330 pF for areas that can only be touched from the inside of the vehicle.

- C = 150 pF for areas that can only be touched from the outside of the vehicle.

Pass criteria:

± 4 / 8 KV: FSC A for severity indices 1 and 2, FSC B for severity index 3

± 15 / (25)1) KV: FSC B for severity indices 1 and 2, FSC C for severity index 3

#### Pre damage ESD

Verification method	VM 54523319-102 v2

This test is intended to check the immunity of components to electrostatic discharges produced:

- by operators during storage, handling, assembly and maintenance. Unpowered test is required.

- by occupants in or near the vehicle and customer service operation. Powered test is required.

The electrostatic discharge immunity test procedure shall be conducted using ISO 10605 with the following conditions and requirements.

| Tests | Direct contact Discharge sequence | Air contact Discharge sequence | Specifications |
| --- | --- | --- | --- |
| Component test: Unpowered tests (handling) | ± 4 / 8 KV On each connector pin2) and conductive surface of the device (component) / system | ± 4 / 8 / 15 / (30)1) KV On all accessible points and other surfaces of the device (component) / system | Supplier test Sequence with a series of 10 discharges for each negative and positive level (from the lowest level to the highest level) Intervals between each discharge: 5s minimum Network: R = 330 Ω, C = 150 pF |
|  | Requirements: FCS A AFTER RECONNECTION |  |  |

- Only for pyrotechnic components.

- For testing pins in contact mode, an extension line for easy discharge can be used.

- The line shall be no longer than 25 mm.

Pass criteria:

No notisable damage

### Immunity to low-frequency magnetic fields (If applicable)

HWC requirement	REQ 54523319-54 v3

From 515-0003 9.1

Scope: This requirement serves to protect systems from interference from power lines, their harmonics

and DC fields present at certain industrial plants. Conforms to ISO 11452-8.

The following requirements are applicable to magnetic fields of zero and low frequencies from 15 Hz up to 150 kHz.

#### Immunity to low-frequency magnetic fields (If applicable)

Verification method	VM 54523319-103 v3

Test levels:

DC field : 25 mT

AC field (15 Hz to 150 kHz): according to ISO 11452-8, test level IV (internal field)

The frequency steps shall be at least 10/decade, linear or logarithmic. If any resonance frequency of the

system can be suspected, the number of steps shall be increased to at least 80 steps/decade,

algorithmic steps, or 3 % step size.

Pass criteria:

System with SI 1: FSC A must be maintained throughout the test.

System with SI 2 or SI 3: At least FSC C must be maintained.

### Emission of low-frequency magnetic fields

HWC requirement	REQ 54523319-55 v2

From 515-0003 9.2

#### Test setup complete vehicle

Verification Method	VM 54523319-104 v2

This test will be performed by Volvo.

The magnetic field shall be measured in accordance with the following test set-up.

The following conditions shall be fulfilled:

- Headlights on

- Radio turned on

- Fan on half speed

- Cab lights turned on

The measurements shall be carried out both at idle and at 2000 rpm, driver and passenger side. The measurement points shall be in accordance with fig. 15, except the bunk location.

Fig. 15

Measurement shall also be carried out at measurement point 6 with the engine shut off.

Pass criteria:

Frequency range 	RMS value (must never be exceeded)

5 Hz - 2 kHz 		2,0 µT

2 kHz - 400 kHz		0,25 µT

## Network/communication

TR information	INF 54523319-35 v2

Not applicable for any component not connecting to a Volvo network or communications bus.

## Diagnostics and control

TR information	INF 54523319-36 v2

The aurora system should support component-level diagnostics and report any failure to Volvo system.

### Diagnostics – Design review

Verification method	VM 54523319-105 v2

To be reviewed with stakeholders.

Pass criteria

Documented/stated compliance with requirements.

# Manufacturing and logistics

## Part handling by GTO/AM

TR information	INF 54523319-37 v2

If any of the COTS components are dropped during manufacturing or aftermarket, the part shall not be installed and shall be returned to Aurora. This is a requirement towards Aurora to make sure this information is added to the PHR (Part Handling Request).

## Component design for assembly

HWC requirement	REQ 54523319-56 v2

The component shall support production requirements on safety and ergonomics:

- Be free from sharp edges

- Secure hand insertion force (as shown in chart below)

- Component design shall allow visibility of assembly operations, no hidden assembly allowed

- Component and fastener design shall allow tightening tool accessibility

- Be easy to attach to a fixture, if applicable (assembly / control fixture)

- Secure clearance between connectors for hand accessibility (30mm)

- The use of glue to assemble parts in Volvo plant shall be prohibited

- Sticking with tape shall be restricted but is acceptable IF conditions are well documented (for ex film antenna or toll collect equipment sticking on windscreen, shall have indication marks on the windscreen)

The component shall support production requirements on industrial capacity and flexibility and in handling and quality:

- Have a minimum of materials, variants and part numbers

- Process emissions and substance usage shall be designed within regulatory and Volvo requirements Do not design a product that requires a process that includes hazardous waste in our factories (eg, solvents, wipes, fill fluids,…)

- Module assembly and sub assembly shall be possible without flipping the module during assembly

- Minimize exposed fiberglass or other substances causing irritation to operator skin, eyes, or respiratory tract.

- Support shortest possible assembly time by minimizing fasteners and implementing quick connect solutions

(e.g : signal, power supply, ground, coolant pipes)

- Support standard tools by using Volvo global fasteners

- Require a minimum of assembly tools by using same fasteners and torque

- Provide tool points for lifting tool

- Ease installation, interface connections and secure quality by supporting:

  - One motion installation and connection (no twist)

  - Self-locating installation (before tightening)

  - Color marks

  - Poka-Yoke

  - No gap and flush, alignment adjustment

  - Connector sensory (e.g: haptic, sound) feedback and locking system

  - No tool required for electrical connection

- Support standalone (i.e: without gateway) programming and diagnostic via manufacturing Off-board diagnostic tool.

- All critical characteristics assembly shall be identified

- No specific ESD equipment (e.g : ESD shoes, floor mats, gloves,…) required when handling the component

- Connector locking only when 100% electrical connected with well-defined visual and audio “click” feedback (haptic feedback)

- No tool shall be required for electrical connection. If a ring terminal interface is used a lock rotation is required.

### Component design for assembly – Design review

Verification method	VM 54523319-106 v2

The fulfilment of assembly requirements shall be discussed and approved before B-release.

Fulfilment of the requirements shall be documented.

The following is required before B-release:

The supplier shall provide Volvo with a description of the mounting interface, including dimensions and tolerances.

The supplier shall provide Volvo with complete installation requirements and, if any, limitation of use for the component in relation to Volvo requirements.

Pass criteria

Documented/stated compliance with requirements.

## Component design for logistics

HWC requirement	REQ 54523319-57 v2

The component shall fulfil the following design-for-logistics requirements:

- The component shall fit into Volvo standard packaging (V-EMB) according to the V-EMB design boxes matrix.

- If the component is larger than Volvo standard packaging (V-EMB), it shall fit into standard transport carriers.

- The component design shall be optimized in order to increase and maximize the filling rate in transport packaging.

- The component shall be optimized for transportation and handling prior to assembly:

  - For ex: Protection plug (for hydraulic holes, or for fragile lens)?

  - For ex: Self protecting case to ease component manipulating and reduce damages, if relevant (minimizing trash)

- No or simplest protection shall be needed during transportation

- When requested by GTO, new part shall be marked with 2D code according to Volvo STD 103-0013. GTO marking requests will be communicated on SL1, SL2 and Component level before CG.

- No incoming part / emballage with dimension exceeding standard trailer interior dimension of 89" width

### Component design for logistics – Design review

Verification method	VM 54523319-107 v2

The component design-for-logistics review shall be initiated and discussed before Concept Gate and verified/approved with design-for-logistics stakeholders before B-release.

Pass criteria

Documented/stated compliance with requirements.

The fulfilment of requirements shall be documented in the Design for Logistics Conclusion Report.

# Aftermarket

## Design for aftermarket

TR information	INF 54523319-38 v2

A new part is backward-compatible if it can replace preceding parts in all previous applications. If Aurora components have any effect on volvo components, it should be coordinated with any other project workgroups focusing on this area.

HWC requirement	REQ 54523319-58 v2

Backward-compatibility solution

[Requirement To Be Defined]

Spare part assortment

[Requirement To Be Defined]

### Design for aftermarket – Design review

Verification method	VM 54523319-108 v2

Fulfilment of the requirements is verified by a DFaM (Design For aftermarket) review.

Pass criteria

Documented/stated compliance with requirements.

# Cybersecurity

## Cybersecurity Engineering

HWC requirement	REQ 54523319-59 v2

If any of the COTS components provided by Aurora are subject to cybersecurity concerns or tampering, then Aurora shall supply a TARA describing the threats and recommended actions for the vehicle platform.

### Cybersecurity engineering – Design review

Verification method	VM 54523319-109 v2

Verified against IEC 62443-3-3, or as agreed within the program Cybersecurity workgroup.

RTS to state the method of verification.

## Cybersecurity supply chain management

HWC requirement	REQ 54523319-60 v2

A software bill of material shall be provided for any component in the assembly containing software.

The required artefact shall include all software components (proprietary and open source), open source licenses, and dependencies included in the delivered hardware component.

### Cybersecurity engineering - Design review

Verification method	VM 54523319-110 v2

Software bill of material to be shared with the program cybersecurity workgroup.

# Other requirement areas

## Marking

HWC requirement	REQ 54523319-61 v2

The component shall be marked with (at a minimum) the following information for identification and manufacturing (GTO) usage.

All information shall be on the same label, applied to the component itself (or wire harness pigtail, if not practical on the component itself).

a. Component Identification marking, according to Volvo STD 5051,16:

MET code: 4.4 – 4G – HPT

b. Barcode (with human-readable text of same information, adjacent), according to Volvo STD 103-0013:

2D DATA MATRIX – MET A1 – P#S#

Additional (supplier-specific) human-readable information is allowable, after discussion with Volvo.

Additional (supplier-specific) machine-readable information in the Data Matrix is NOT allowed, without discussion with Volvo.

Any branding in plastic or metal parts are to be approved by the Volvo Autonomous Solutions commercial team.

Any required legal markings (e.g., certifications) are under the responsibility of the supplier.

### Marking – Design review

Verification method	VM 54523319-110 v2

The need for marking shall be identified during RTS.

Marking shall be clearly indicated in the design-engineering documentation (drawings), in accordance with the applicable STD (see requirement) prior to C-release.

Pass criteria

Documented/stated compliance with requirements.

# References

## Reference documents

TR information	INF 54523319-39 v2

The following table lists all documents that this TR refers to.

| Document | Description | Issue/ Version |
| --- | --- | --- |
| Volvo STD 100-0002 | Chemical substances which must not be present in processes or chemical products within the Volvo Group | 16 |
| Volvo STD 100-0003 | Chemical substances which should not be present in processes or chemical products within the Volvo Group | 19 |
| Volvo STD 100-0005 | Chemical substances which must not be present in products within the Volvo Group | 10 |
| Volvo STD 100-0006 | MDS (Material Data Sheet) reporting of substances and material composition | 8 |
| Volvo STD 400-0001 | Standard atmospheres for conditioning and testing | 1 |
| Volvo STD 103-0002 | Material and designations | 3 |
| Volvo STD 103-0010 | Marking of aluminum parts | 4 |
| Volvo STD 104-0001 | Flammability of interior materials | 3 |
| Volvo STD 5051,16 | Marking | 17 |
| Volvo STD 423-0030 | Scratch resistance, Erichsen pen | 1 |
| Volvo STD 423-0040 | Scratch resistance using abrasive paper | 1 |
| Volvo STD 1024,7132 | Chip resistance | 6 |
| Volvo STD 525-0001 | Wires - Dimensions, test methods and requirements | 2 |
| Volvo STD 525-0004 | Cables - Dimensions, test methods and requirements | 1 |
| Volvo STD 1026,8202 | Grey scale – staining | 4 |
| Volvo STD 1027,337 | Light exposure - Accelerated ageing of exterior material | 3 |
| Volvo STD 515-0003 | EMC requirements | 5 |
| Volvo STD 515-0004 | Sealed temperature/humidity cyclic test | 1 |
| Volvo STD 411-0001 | Plastics | 2 |
| Volvo STD 105-0007 | Special characteristics - Definition and application | 3 |
| Volvo STD 101-0005 | Symbols, designations and general drawing methods - Basic rules for representation | 19 |
| Volvo STD 910-0005 | Supplier parts and supplier documentation | 4 |
| Volvo STD 103-0013 | Parts marking with bar codes and 2d symbologies - Marking of parts | 7 |

| Document | Description | Issue/ Version |
| --- | --- | --- |
| ISO 16750-1: 2006 ISO 16750-2: 2012 ISO 16750-3: 2013 ISO 16750-4: 2010 ISO 16750-5: 2010 | Road vehicles – Environmental conditions and testing for electrical and electronic equipment | 2 4 3 3 2 |
| ISO 20653 | Degrees of protection | 2 |
| UL94V | Fire resistance | 6 |
| EMC:UN/ECE R10.06. | E-marking |  |
| IPC J-STD-001 | Requirements for Soldered Electrical and Electronic Assemblies | E |
| IPC J-STD-004 | Requirements for Soldering Fluxes | XX |
| IEC 60068-2-14 | Test N: Change of temperature | 2009 |
| IEC 60068-2-38 Z/AD | Test Z/AD: Composite temperature/humidity cyclic test | 2009 |
| IEC 60068-2-78 | Damp heat, steady state | 2012 |
| SAE J2721 | Recommended Corrosion Test Methods For Commercial Vehicle Components | 2011 |
| TR 20808076 | General TR for connectors | 4 |
| TR 53180821 | ADM Aurora TR | 1 |

## Abbreviations and terms

TR information	INF 54523319-40 v2

The following table lists important abbreviations that are used in the TR.

| Abbreviation/ Term | Description |
| --- | --- |
| 2-D | Two-dimensional |
| 3-D | Three-dimensional |
| A | Ampere, current |
| ADM | Aurora Driving Module |
| AET | Accelerated Endurance Test |
| CAD | Computer-Aided Design |
| CC | Critical Characteristics |
| CEM3 | Low cost 2 glass fiber woven layers + non-woven fiber core substrate |
| Component | Depending on context, either the complete device for which this TR is written (i.e., the XXXX), or (any or all of) its constituent subcomponents. |
| CTE | Coefficient of thermal Expansion |
| DfR | Design for Recycling |
| DUT | Device Under Test |
| ECU | Electronic Control Unit |
| EMC | Electromagnetic Compatibility |
| ENIG | Electro-less Nickel Gold PCB surface treatment |
| FMEA | Failure Mode Effect Analysis |
| FR4 | Glass fiber printed circuit substrate (7 woven glass fiber layers or more) |
| FSC | Functional Status Classification |
| h | Hour |
| HASL | Hot Air solder leveling PCB surface treatment |
| HW | Hardware |
| I/O | Input/Output |
| ICD | Interface Control Document |
| ImAg | Immersion silver PCb surface treatment |
| IMDS | International Material Data Sheet, (a material data system for automotive OEMs) |
| ImSn | Immersion Tin PCB surface treatment |
| INF | Information |
| IP | Ingress Protection |
| IPC | PCB and assembly industry standard organization (www.ipc.org) |
| ISO | International organization for standardization |
| MET | Method |
| N/A | Not Applicable |
| NRP | Near Range Pod |
| ODD | Operation Design Domain |
| OSP | Organic Surface Protection. PCB surface solder ability preservation |
| PCB | Printed Circuit Board |
| PPAP | Production Part Approval Process |
| ppb | Parts per billion |
| QI | Quality Impression |
| REQ | Requirement |
| RTS | Review of Technical Specification |
| SC | Short Circuit |
| SC | Significant Characteristics |
| SMD | Surface-Mounted Device |
| STD | Standard |
| SVAT | Standard Vehicle Autonomous Transport |
| SW | Software |
| T | Temperature degree C |
| Ta | Time Application |
| TBC | To be Confirmed |
| TBD | To be Decided |
| Tmax | Maximum ambient temperature |
| Tmin | Minimum ambient temperature |
| Tnom | Nominal ambient (room) temperature |
| TR | Technical Requirements |
| TRA | Test Requirements Analysis |
| Usmax | Maximum operating supply voltage |
| Usmix | Minimum operating supply voltage |
| Usnom | Nominal operating supply voltage |
| V | Volt |
| Vcc | Low voltage regulated component supply |
| VM | Verification method |
| Vreg | General voltage regulated component supply output |
| Vs | General component power supply voltage connection connector pin(s). From any source |

# Changelog

## New items

## Updated items

## Deleted items

# Approved Deviations